CYCLOPS PRO

Full System Block Diagram, Pin Assignments & STM32N657 ICD

K-band FMCW DDM-MIMO Radar + Dual-Band Polarimetric EMVS Upgrade

24.125 GHz + 10 GHz | STM32N657 + ADAR2001 + ADAR2004 x2 + AD7380-4 x3 + LMX2594

HW-RDR-001

Rev C  DRAFT  (integrated block diagram + ICD)

2026-04-20

Ocupoint   |   CONFIDENTIAL

# Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| A | 2026-04-20 | M. Orr / Claude | Initial release -- full-system block diagram for Cyclops Pro consolidating CYC-SYS-002 Rev 1.0 DualTX baseline + EMVS dual-band upgrade. |
| B | 2026-04-20 | M. Orr / Claude | ADF5904 consolidation -- 2 spare RX channels absorb both EMVS cells' K-band (Ch C/D). ADF5904 #2/#3 + ADRF5020 SP2T eliminated. ~−$16/board BOM savings; asymmetric IF routing (Ch A/B → AD7380-4 (reassigned to #3 in Rev E), Ch C/D → STM32 internal ADC). |
| C | 2026-04-20 | M. Orr / Claude | Integrated HW-ICD-001 as §7–§12. Adds STM32N657 master ICD (every MCU pin), per-component pin tables for every RF/mixed-signal part, net-naming convention, power-domain summary, STM32 peripheral allocation, validation checklist. Supersedes HW-ICD-001 Rev A standalone. §1–§6 block-diagram content unchanged from Rev B. |
| D | 2026-04-21 | M. Orr / Claude | EMVS front-end redesign: ADRF5024 SP4T + HMC753 LNA + HMC1065 mixer replaced by ADAR2004 ×2 (4-ch integrated Rx mixer, 10–40 GHz) + AD7380-4 ×2 (quad 16-bit SAR, 4 MSPS simultaneous) + LMX2594 (dedicated 6 GHz LO PLL). Lower band moved 9.5→10 GHz. Polarimetric dwell collapses 4 CPI→1 CPI. ADF5904 Ch A/B remapped to AD7380-4 #3 (CH9/CH10). Ch C/D unused.. STM32 internal ADC freed. 20 dB coupler removed. Net BOM savings ~$37/board at qty 1. |
| E | 2026-04-29 | M. Orr / Claude | ADAR2001 EMVS TX integration: adds ADAR2001 (10-40 GHz, 4-ch TX with x4 multiplier) driving TX-EMVS 1 cell (Ex/Ey/Hx/Hy). Replaces both bow-tie antennas. Full polarimetric TX diversity. Eliminates X-band TX gap present since Rev B. SPI6 (RF_SPI) shared bus; CS via MAX7301 I/O expander port P20 (on analog board). ADAR2004 MADV/MRST shared with ADAR2001 for synchronized frequency stepping. Pin 50 (RxRST/PA10) repurposed as ADAR2001 TxADV; RxRST function relocated to MAX7301 P21. Digital board unchanged; RF board ECO. Analog board mating connector ICD added (§13). |

# Document Information

| Document Number | HW-RDR-001 |
| --- | --- |
| Title | Cyclops ProFull System Block Diagram, Pin Assignments & STM32N657 ICD |
| Revision | C (Draft)integrated |
| Classification | CONFIDENTIALOcupoint Internal |
| Status | DRAFTblock diagram sections engineer-validated; pin assignments PROPOSED pending schematic review |
| STM32 assumed | STM32N657X0H3Q, LQFP144 package, Nucleo-144 MB1940 reference |
| Supersedes | HW-RDR-001 Rev A/B/C/D. Rev E adds ADAR2001 wideband EMVS TX (10-40 GHz). Rev D replaced EMVS RX front-end with ADAR2004+AD7380-4+LMX2594. |
| Prepared by | M. Orr, Ocupoint Engineering |
| Reference documents | • CYC-SYS-002 Rev 1.0Cyclops Dual TX System Design (2026-03-30) • Cyclops Pro Hardware Reference (2026-04-14) • cyclops_dual_band_emvs.docx -- Dual-Band Polarimetric Antenna Upgrade |

# 1.  Purpose & Scope

This document is the controlled master reference for the Cyclops Pro radar platform. It consolidates three previously separate views into a single authoritative source:

- The K-band DualTX FMCW DDM-MIMO baseline and the dual-band polarimetric EMVS RX upgrade, merged into one system view

- The full STM32N657 interface control document listing every MCU pin that terminates on a radar-board net

- Per-component pin assignments for every RF and mixed-signal part, with net names consistent with the master ICD

§2–§6 are engineer-validated. §7–§12 (pin assignments and ICD) are PROPOSEDthey reflect engineering judgment against vendor datasheets but have not yet been cross-checked against the Nucleo-144 MB1940 schematic or against the STM32N657 alternate-function matrix for conflicts. See §12 for the open-items checklist.

## 1.1  Configuration shown

- DualTX K-band baseline -- 2× ADF4159 chirp PLLs, 2× ADF5901 TX VCO/PA, 2× HMC1131 MPA, 2× PMA3-24323LN+ RX LNA, 1× ADF5904 (Ch A/B main beams → AD7380-4 #3 via CH9/CH10 (Rev E remap); Ch C/D unused)

- Integrated EMVS upgrade (Rev E)TX-EMVS 1 cell (4-ch: Ex, Ey, Hx, Hy, 10-40 GHz) placed top-left adjacent to TX1 via wall, 1× ADAR2001 4-ch TX with x4 multiplier (10-40 GHz, +5 dBm, dedicated EMVS transmitter), 2× 4-component EMVS RX cells [Ex, Ey, Hx, Hy], 2× ADAR2004 4-ch Rx mixer (simultaneous, 10-40 GHz), 2× AD7380-4 quad 16-bit SAR ADC (4 MSPS simultaneous), 1× LMX2594 dedicated LO PLL (shared TX/RX via x4 mult). ADAR2001 drives single TX-EMVS 1 cell (Ex, Ey, Hx, Hy) replacing both bow-tie antennas. Full polarimetric TX diversity via per-chirp pol-cycling (TxADV state machine). Phase-coherent with ADAR2004 RX from common LMX2594 source. 1-CPI polarimetric dwell.

- STM32N657X0H3Q compute core running range FFT, Doppler FFT, DDM-MIMO separation, MUSIC/monopulse, CFAR, polarimetric classification, EMVS Poynting-vector elevation DOA, AIC + polarimetric null, Kalman tracking

# 2.  Full System Block Diagram

Figure 1 is the controlled master block diagram. Subsequent sections expand the signal chains, interfaces, DSP pipeline, and pin-level interconnect.

Figure 1.  Cyclops Pro full system block diagram -- Rev E.  ADAR2001 drives TX-EMVS 1 cell (replaces bow-ties).  LMX2594 RFoutA → Wilkinson → ADAR2004 x2 RX; RFoutB direct → ADAR2001 TX.  Board placement: TX1|via|TX-EMVS 1 (top-left), RX-EMVS 1 (top-right), RX-EMVS 2 (bottom-left), TX2 (bottom-right). Analog board connector ICD in §13.

# 3.  Signal Chain Summary

## 3.1  Transmit chain (×2 paths, dual-band)

| Stage | Part | Gain / Power | Band(s) | Notes |
| --- | --- | --- | --- | --- |
| Chirp PLL | ADF4159 (×2) | — | 13 GHz output | TXDATA → DDM π-phase code; BPSK comms via BSS138 gate |
| VCO + TX PA | ADF5901 (×2) | +8 dBm out | 24.0–24.25 GHz | Internal 2× doubler; LO (#1 only) feeds ADF5904 #1 |
| MPA | HMC1131 (×2) | +22 dB, P1dB +24 | 24–35 GHz | GaAs pHEMT, thermal-via-array required |
| TX antenna | TX-EMVS 1 cell | ~1 dBi/elem | 10–40 GHz | TX-EMVS 1 (4-ch: Ex, Ey, Hx, Hy), ADAR2001 quad PA, pol-cycled via TxADV state machine |
| EIRP (K) | Total | +36 dBm (4 W) | 24.125 GHz | Primary radar transmit |

## 3.2  Main receive chain (×2 beams, K-band)

| Stage | Part | Gain / NF | Cumulative NF | Notes |
| --- | --- | --- | --- | --- |
| RX antenna | 7-elem K-band patch (×2) | +6 dBi | — | SA2 = Beam 1, SA3 = Beam 2 |
| LNA | PMA3-24323LN+ (×2) | +16 dB, NF 3.1 | 3.1 dB | Sets main-RX NF |
| RX mixer+IF | ADF5904 #1 (Ch A/B used) | +22 dB, NF ~10 | 3.35 dB | Ch A/B ← PMA3; Ch C/D FREE (Ch A/B → CH9/CH10 → ADC3. Ch C/D unused.) |
| ADC (main) | AD7380-4 #1 | 16-bit @ 4 MSPS | 3.35 dB | ADF5904 Ch A/B → AD7380-4 #3 via CH9/CH10 (analog board remap). No AAF needed. |

## 3.3  EMVS polarimetric receive chain (×2 cells, integrated)

Rev E adds the ADAR2001 as a dedicated EMVS wideband transmitter (10-40 GHz), eliminating the X-band TX gap present in all prior revisions. LMX2594 RFoutA feeds ADAR2004 ×2 via 2-way Wilkinson (+3.5 dBm/port); RFoutB feeds ADAR2001 directly (+7 dBm). Both outputs derive from the same on-chip VCO — inherently phase-coherent for FMCW dechirp. No 3-way splitter required. Prior SP4T-muxed time-division RX architecture (Rev C) and coupler-tap TX (Rev B/C) fully eliminated. 1-CPI polarimetric dwell.

| Stage | Part | Qty | Function | Notes |
| --- | --- | --- | --- | --- |
| EMVS cell | PCB copper | 2 | [Ex Ey Hx Hy] 4-component | 3.16×3.16 mm, 8 vias. TX-EMVS 1 identical fabrication. |
| 4-ch Rx mixer | ADAR2004ACCZ | 2 | Simultaneous 4-ch LNA+mixer | 10–40 GHz, ×4 LO mult, SPI seq, 48-LGA, 910 mW |
| Quad ADC | AD7380-4BCPZ | 2 | 4 simultaneous diff. channels | 16-bit, 4 MSPS, ext ref 2.5 V, 4×4 mm LFCSP-24 |
| LO synth | LMX2594RHAR | 1 | Shared 6 GHz PLL+VCO | ×4 in ADAR2004 → 24 GHz; via LMX2594 RFoutA → 2-way Wilkinson (ADAR2004 #1 + #2); RFoutB → ADAR2001 direct splitter |
| LO splitter | TCP-2 class | 1 | LMX2594 RFoutA → 2-way Wilkinson (ADAR2004 #1 + #2); RFoutB → ADAR2001 direct | RFoutA: +3.5 dBm/port to ADAR2004 ×2; RFoutB: +7 dBm direct to ADAR2001 |
| Ref voltage | ADR4525 class | 1 | Shared 2.5 V reference | For both AD7380-4 REFIN pins |

Key architectural advantages (Rev E): simultaneous 4-channel TX and RX (full Mueller matrix per CPI), 1-CPI polarimetric dwell, adaptive TX mode (pol-cycled ↔ 4-ch coherent via TxADV), dedicated wideband LO with phase-coherent TX/RX, native 3.3 V SPI, all prior SP4T/coupler/bow-tie components eliminated.

# 4.  Key System Interfaces

High-level interface contracts. Pin-level detail in §7.

## 4.1  Clock & Reference

| Signal | Source | Destination | Notes |
| --- | --- | --- | --- |
| 100 MHz TCXO | Primary OSC | ADF4159 ×2, ADF5901 ×2, ADF5904 #1, LMX2594 | Shared reference for phase coherence |
| 24 GHz LO (main) | ADF5901 #1 LO_OUT | ADF5904 #1 (on-die fanout to Ch A/B) | -1 dBm; Ch C/D now unused |
| 6 GHz LO (EMVS) | LMX2594 | ADAR2004 #1 + #2 (via 2-way splitter) | +3.5 dBm/port; ×4 on-die → 24 GHz |
| 2.5 V ref | ADR4525 class | AD7380-4 #1 + #2 REFIN | Shared external; ≤10 ppm/°C |

## 4.2  Chirp & ADC timing

| Function | Peripheral | Net | Notes |
| --- | --- | --- | --- |
| Chirp ramp trigger | TIM1_CH2 (PE11) | CHIRP_START → ADF4159 TRIG | 28.6 kHz, per 35 µs chirp |
| ADC convert | TIM1_CH1 (PE9) | CNV_MASTER → AD7380-4 ×3 CS | 4 MSPS, CS-triggered |
| ADC capture (main) | SPI + GPDMA | AD7380-4 #1 SDOA/B | Shared SPI bus, 2 of 4 ch used |
| EMVS ADC capture | SPI + GPDMA | AD7380-4 #2/#3 SDOA-D | 4-wire mode, full-rate readout |
| CPI sync | Software | — | 8.96 ms CPI; 1-CPI polarimetric dwell (down from 4-CPI) |

## 4.3  Control plane (SPI + GPIO)

| Bus / GPIO | Width | Devices / Function | Notes |
| --- | --- | --- | --- |
| SPI6 (RF_SPI) | SCK/MOSI/MISO + 6 CS | ADF4159 ×2, ADF5901 ×2, ADF5904, LMX2594 | RF config bus via SPI6 + MAX7301 I/O expander (ADAR2001 + ADAR2004 ×2 + LMX2594) |
| SPI4/5 (ADC shared) | SCK/MOSI/MISO + 3 CS | AD7380-4 ×3 (main + EMVS ×2) | All ADCs on one bus, CS-select |
| TXDATA | 2 lines | ADF4159 #1/#2 TXDATA | π-per-chirp DDM / BPSK |
| BPSK gate | 2 lines | BSS138 per ADF4159 | HIGH widens PLL BW for comms |
| Master enables | 5 lines | TCXO/PLL/TX/RX/EMVS EN | Sequencing with delays |
| Lock detect | 3 lines | ADF4159 ×2 MUXOUT + LMX2594 MUXout | Input; low on loss |

## 4.4  SPI Bus Architecture & Connection Strategy

The Cyclops Pro uses two SPI buses with fundamentally different usage patterns. SPI6 (RF_SPI) is a low-speed configuration bus touched only at power-up and mode changes. SPI4/5 is a high-speed data bus running continuously during radar operation. The two buses never contend because they operate in different phases of the system lifecycle.

BUS 1: SPI6 (RF_SPI) -- RF Configuration (shared, low-speed)

SCK = PA5, MOSI = PA7, MISO = PA6. Clock speed 1-10 MHz. Standard SPI Mode 0 (CPOL=0, CPHA=0).

| Device | CS Pin | When Accessed | Data Direction | Purpose |
| --- | --- | --- | --- | --- |
| ADF4159 #1 | PB12 | Init + mode changes | Write-only (readback via MUXOUT) | Chirp PLL register load: ramp slope, bandwidth, deviation, TXDATA source. ~20 registers at power-up. Re-written only on waveform parameter change. |
| ADF4159 #2 | PB13 | Init + mode changes | Write-only | Identical register set to #1. Must be programmed after #1 for phase coherence. |
| ADF5901 #1 | PB14 | Init only | Write-only | VCO calibration, TX PA enable, output power, LO output enable. Written once at power-up (~5 ms), never touched during operation. |
| ADF5901 #2 | PB15 | Init only | Write-only | Same as #1. LO output disabled (not used on TX2 path). |
| ADF5904 #1 | PD8 | Init only | Write + readback | Channel enable mask, gain trim, temperature sensor read. Written once at power-up. |
| ADAR2004 #1 | PF0 | Init only | Write-only | Channel enable, LNA bias, mixer config, SPI sequencer pattern. Programmed once; sequencer runs autonomously during operation. |
| ADAR2001 #1 | MAX7301 P20 | Init only | Write-only | Rev E: EMVS wideband TX. x4 multiplier band, PA channel, attenuator. CS via MAX7301 I/O expander port P20 on analog board (SPI6/RF_SPI bus). No direct GPIO consumed. |
| ADAR2004 #2 | PF1 | Init only | Write-only | Same register set as #1. Both cells configured identically. |
| LMX2594 | PF2 | Init only | Write + readback | PLL N-divider, charge pump, VCO cal, output divider, MUXout config. ~113 registers. Lock verified via MUXout (PF3) before enabling EMVS. |

After the power-up initialization sequence (~50 ms total including PLL lock times), SPI6 is idle during normal radar operation. The firmware may re-access SPI6 to change chirp parameters between scan modes, but this happens between CPIs, never during a chirp.

BUS 2: SPI4/5 -- ADC Data (shared, high-speed)

SCK, MOSI (SDI), MISO (SDOA) on SPI4/5 peripheral pins (exact AF TBD per STM32N657 CubeMX verification). Clock speed up to 80 MHz. CS falling edge triggers simultaneous conversion on all 4 channels of the addressed AD7380-4.

| Device | CS Pin | When Accessed | Data Direction | Purpose |
| --- | --- | --- | --- | --- |
| AD7380-4 #1 | PF6 | Every sample (4 MSPS) | Read (4-wire: SDOA/B/C/D) | EMVS Cell 2 (all 4 channels): ADAR2004 #2 Ex/Ey on AINA/AINB (CH1/CH2), Hx/Hy on AINC/AIND (CH3/CH4). Analog board remap — all Cell 2 IF on one ADC, clean DMA pipeline. |
| AD7380-4 #2 | PF4 | Every sample (4 MSPS) | Read (4-wire: SDOA/B/C/D) | EMVS Cell 1. ADAR2004 #1 IF outputs on all 4 channels (Ex, Ey, Hx, Hy). |
| AD7380-4 #3 | PF5 | Every sample (4 MSPS) | Read (4-wire: SDOA/B/C/D) | Main RX beam: ADF5904 Ch A/B on AINA/AINB (CH9/CH10). AINC/AIND unused. Analog board remap — main beam on dedicated ADC, no mixed sources. |

The AD7380-4 supports 4-wire simultaneous readout mode: when CS falls, all 4 channels sample simultaneously, then SDOA/SDOB/SDOC/SDOD clock out in parallel on the rising SCLK edges. At 80 MHz SCLK, a 16-bit readout completes in 200 ns -- well within the 250 ns sample period at 4 MSPS.

SPI TIMING WITHIN A CHIRP

Each 35 us chirp contains ~140 ADC samples at 4 MSPS. The sampling sequence per sample period (250 ns) is:

| Phase | Duration | Action |
| --- | --- | --- |
| 1. CS assert #1 | ~10 ns | Pull PF6 low -- triggers AD7380-4 #1 conversion (EMVS Cell 2) |
| 2. Convert | ~125 ns | AD7380-4 #1 converts all 4 channels simultaneously |
| 3. Readout #1 | ~200 ns | Clock out 16 bits on SDOA+SDOB (Ch A/B data) via GPDMA |
| 4. CS deassert #1 | ~10 ns | Pull PF6 high |
| 5. CS assert #2 | ~10 ns | Pull PF4 low -- triggers AD7380-4 #2 conversion (EMVS Cell 1) |
| 6-7. Convert + read #2 | ~325 ns | Same sequence for Cell 1 |
| 8-10. CS #3 cycle | ~335 ns | Same for AD7380-4 #3 (Main RX beams) |

Total SPI time per sample: ~900 ns. At 250 ns sample period, the three ADCs must be time-interleaved across consecutive sample slots, or the EMVS ADCs can run at a submultiple (e.g. 2 MSPS each while main runs at 4 MSPS). The exact interleaving strategy depends on whether all three ADCs share a single SPI peripheral or use separate peripherals with DMA. This is a firmware architecture decision documented in the bring-up plan.

POWER-UP SEQUENCING

| Step | Time | SPI Action | Notes |
| --- | --- | --- | --- |
| 1. TCXO enable | PE3 HIGH | None | 10 ms settle time for 100 MHz reference |
| 2. PLL_EN | PE4 HIGH | None | Powers ADF4159 CE, ADF5904 CE |
| 3. Program ADF4159 x2 | SPI6 | ~40 register writes | Chirp parameters, ramp config |
| 4. Program ADF5901 x2 | SPI6 | ~10 register writes | VCO cal, PA enable, LO config |
| 5. Program ADF5904 #1 | SPI6 | ~8 register writes | Channel enable, gain trim |
| 6. TX_EN | PE0 HIGH | None | Powers HMC1131 + ADF5901. Wait 1 ms for bias settle. |
| 7. RX_EN | PE1 HIGH | None | Powers PMA3 + ADF5904 analog. |
| 8. Program LMX2594 | SPI6 | ~113 register writes | 6 GHz PLL config. Verify lock on PF3 MUXout. |
| 9. Program ADAR2001 + ADAR2004 x2 | SPI6 | ~20 register writes each | ADAR2001 (CS via MAX7301 P20): x4 mult band, PA channel, attenuator. ADAR2004 x2 (CS via P9/P10): channel config, sequencer pattern |
| 10. EMVS_EN | PE2 HIGH | None | Powers ADAR2004/AD7380-4/LMX2594 analog. Wait 1 ms. |
| 11. Verify all locks | Read | ADF4159 LD (PD4/PD5) + LMX2594 LD (PF3) | All high = system ready |
| 12. Begin chirping | TIM1 start | ADC SPI begins | CHIRP_START + CNV_MASTER active |

Total power-up to first chirp: ~50 ms (dominated by PLL lock times and TCXO settle). SPI6 is idle after step 9. SPI4/5 begins continuous operation at step 12.

# 5.  Signal Processing Pipeline (STM32N657)

Cortex-M55 main thread + Ethos-U65 NPU (parallel polarimetric classifier). Latency budget: 2.1 ms per CPI.

| # | Stage | Input | Output | Notes |
| --- | --- | --- | --- | --- |
| 1 | Range FFT (Taylor −35 dB) | Time-domain IQ/chirp | Range spectrum | 2048-pt main, 128-pt EMVS |
| 2 | Doppler FFT + Hamming | Range × chirps | Range-Doppler map | 128 chirps/CPI |
| 3 | DDM-MIMO separation | R-D combined TX1+TX2 | Split TX channels | π-per-chirp phase decode |
| 4 | Virtual array | Per-TX × per-RX | 4-element virtual | 2 TX × 2 RX |
| 5 | MUSIC / monopulse | Virtual array | Azimuth θ | Covariance / Σ-Δ |
| 6 | SO-CFAR | R-D magnitude | Detection list | Range-adaptive α, Doppler gating |
| 7 | Polarimetric classifier [NPU] | S_HH, S_HV | Human/wall flag | ¦S_HV¦/¦S_HH¦ |
| 8 | EMVS DOA (S_z) | [Ex Ey Hx Hy] × 1 CPI | Elevation θ_el | Ex·Hy* − Ey·Hx* (simultaneous) |
| 9 | Inter-CPI phase correction | Wall reference | LO drift | Applied before pol ratio |
| 10 | AIC + pol null | Detections + flag | Wall-suppressed | 40–55 dB combined |
| 11 | Kalman tracker | (R, θ_az, θ_el, IMU) | 3D track | Pos/vel/uncertainty |

# 6.  References

| Document | Subject | Status at Rev D |
| --- | --- | --- |
| CYC-SYS-002 Rev 1.0 | Cyclops Dual TX System DesignDDM-MIMO architecture, ADF pinmap, link budget | Draft, 2026-03-30 |
| Cyclops Pro Hardware Reference | Master HW refantenna design, MIMO virtual aperture, monopulse, SP pipeline | Working doc, 2026-04-14 |
| cyclops_dual_band_emvs.docx | Dual-band upgrade — TX-EMVS 1 cell + ADAR2001, EMVS RX cells, interface chips, BOM delta | Preliminary, pending HFSS |
| HW-ADC-001 / HW-ADC-002 | AD7380-4 #1 interface + LTC2323 comparison | Finalized |
| HW-BRL-001 Rev B | SiC-CF composite barrel design (peripheral to radar) | Released |
| HW-ICD-001 Rev A | Standalone ICDSUPERSEDED by §7–§12 of this document | Superseded 2026-04-20 |

# 7.  STM32N657 Master ICD

Every STM32N657 pin that connects to a radar-board net. Excludes debug/SWD, USB, Nucleo LEDs/buttons, and unused pins. All ADC interfaces are native 3.3 V (AD7380-4 family). No level shifter required in Rev D.

## 7.1  Reference / Clock

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OSC_IN | PH0 | HSE_IN | STM32_HSE_25MHZ | IN | 25 MHz XTAL (STM32-local) | 3.3 | Not radar; STM32 core clock |
| OSC_OUT | PH1 | HSE_OUT | STM32_HSE_25MHZ | OUT | 25 MHz XTAL | 3.3 | XTAL drive |
| — | — | — | RADAR_REF_100MHZ | — | (not to STM32) TCXO → ADF4159 ×2 + ADF5901 ×2 + ADF5904 #1 + LMX2594 | 1.8 | 100 MHz radar-coherent, NOT to STM32 |

## 7.2  SPI6 (RF_SPI) — RF Configuration Bus

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PA5 | A5 | SPI6_SCK | SPI_SCK | OUT | ADF4159 ×2 CLK, ADF5901 ×2 CLK, ADF5904 #1 CLK | 3.3 | ≤ 10 MHz; matched-length |
| PA7 | A7 | SPI6_MOSI | SPI_MOSI | OUT | ADF4159 ×2 DATA, ADF5901 ×2 DATA, ADF5904 #1 DATA | 3.3 | Shared MOSI; per-dev CS |
| PA6 | A6 | SPI6_MISO | SPI_MISO | IN | ADF4159 ×2 MUXOUT, ADF5904 #1 MUXOUT | 3.3 | Readback; pull-down 100k |

## 7.3  GPIOSPI Chip Selects (active low)

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB12 | B12 | GPIO | CS_ADF4159_1 | OUT | ADF4159 #1 LE | 3.3 | Active low; pull-up 10k |
| PB13 | B13 | GPIO | CS_ADF4159_2 | OUT | ADF4159 #2 LE | 3.3 | Active low; pull-up 10k |
| PB14 | B14 | GPIO | CS_ADF5901_1 | OUT | ADF5901 #1 LE | 3.3 | Active low; pull-up 10k |
| PB15 | B15 | GPIO | CS_ADF5901_2 | OUT | ADF5901 #2 LE | 3.3 | Active low; pull-up 10k |
| PD8 | D8 | GPIO | CS_ADF5904_1 | OUT | ADF5904 #1 LE | 3.3 | Active low; pull-up 10k |

## 7.4  Chirp / BPSK Control

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PE9 | E9 | TIM1_CH1 | CNV_MASTER | OUT | AD7380-4 #1 CNV () + STM32 ADC trigger | 3.3 | 2 MHz, ~14 ns min |
| PE11 | E11 | TIM1_CH2 | CHIRP_START | OUT | ADF4159 #1/#2 TRIG | 3.3 | 28.6 kHz (1/35 µs) |
| PD0 | D0 | GPIO | TXDATA_1 | OUT | ADF4159 #1 TXDATA | 3.3 | Straight-through on TX1 |
| PD1 | D1 | GPIO | TXDATA_2 | OUT | ADF4159 #2 TXDATA | 3.3 | π DDM phase code |
| PD2 | D2 | GPIO | BPSK_GATE_1 | OUT | BSS138 #1 gate | 3.3 | HIGH widens PLL BW for comms |
| PD3 | D3 | GPIO | BPSK_GATE_2 | OUT | BSS138 #2 gate | 3.3 | TX2 path |
| PD4 | D4 | GPIO IN | LD_ADF4159_1 | IN | ADF4159 #1 MUXOUT (LD mode) | 3.3 | Low = loss of lock |
| PD5 | D5 | GPIO IN | LD_ADF4159_2 | IN | ADF4159 #2 MUXOUT (LD mode) | 3.3 | Low = loss of lock |

## 7.5  ADAR2001 + ADAR2004 + LMX2594 SPI Config (via SPI6/RF_SPI + MAX7301)

ADAR2001, ADAR2004 ×2, and LMX2594 share the SPI6 (RF_SPI) bus. All RF chip-select lines are generated locally on the analog board by a MAX7301 28-port I/O expander (CS_IO_EXP on connector pin 26/PA11). CS_ADAR2001 = MAX7301 port P20. ADAR2004 MADV (pin 58/PG13) and MRST (pin 46/PB12) are shared with ADAR2001 on the analog board. Connector pin 50 (PA10, was RxRST) repurposed as ADAR2001 TxADV for per-chirp polarization cycling; RxRST function relocated to MAX7301 P21. See §13 for full analog board connector ICD.

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | F0 | GPIO | CS_ADAR2004_1 | OUT | ADAR2004 #1 CSB | 3.3 | Active low; SPI6 (RF_SPI) via MAX7301 |
| PF1 | F1 | GPIO | CS_ADAR2004_2 | OUT | ADAR2004 #2 CSB | 3.3 | Active low; SPI6 (RF_SPI) via MAX7301 |
| MAX7301 P20 | TBD | GPIO | CS_ADAR2001 | OUT | ADAR2001 #1 CSB | 3.3 | Rev E: CS via MAX7301 P20; SPI6 (RF_SPI) shared bus |
| PF2 | F2 | GPIO | CS_LMX2594 | OUT | LMX2594 CSB | 3.3 | Active low; SPI6 (RF_SPI) via MAX7301 |
| PF3 | F3 | GPIO IN | LD_LMX2594 | IN | LMX2594 MUXout (LD mode) | 3.3 | Low = loss of lock |

## 7.6  AD7380-4 ADC Bus (shared SPI: #1 EMVS Cell 2, #2 EMVS Cell 1, #3 Main Beam)

All three AD7380-4 ADCs share one SPI bus: #1 = EMVS Cell 2, #2 = EMVS Cell 1, #3 = Main Beam. Native 3.3 V — no level shifter. ADC #1 and #2 read 4 channels each; ADC #3 reads 2 channels (AINA/AINB only). with an additional chip-select. All AD7380-4 devices are native 3.3 V -- no level shifter required. Eliminates SPI3, TXS0108E, and 1.8 V digital rail from Rev C.

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (shared) | (shared) | SPI SCK | ADC_SCK | OUT | AD7380-4 #1/#2/#3 SCLK | 3.3 | Shared with EMVS ADCs |
| (shared) | (shared) | SPI MOSI | ADC_SDI | OUT | AD7380-4 #1/#2/#3 SDI | 3.3 | Config writes |
| (shared) | (shared) | SPI MISO | ADC_SDOA | IN | AD7380-4 active SDOA | 3.3 | Muxed via CS |
| PF6 | F6 | GPIO | CS_AD7380_MAIN | OUT | AD7380-4 #1 CS | 3.3 | Active low; triggers conversion |

AD7380-4 #1 captures EMVS Cell 2 (ADAR2004 #2) on all 4 channels: Ex/Ey on AINA/AINB, Hx/Hy on AINC/AIND. Analog board routes ADAR2004 #2 IF outputs to connector CH1-CH4.

## 7.7  AD7380-4 ×2 EMVS ADC (SPI4)

Both AD7380-4 share SPI4 bus with separate chip-selects. 4-wire mode: SDOA/SDOB/SDOC/SDOD output simultaneously for full-rate readout. Native 3.3 Vno level shifter required.

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PE12 | E12 | SPI4_SCK | EMVS_ADC_SCK | OUT | AD7380-4 #1/#2 SCLK | 3.3 | Shared; ≤ 80 MHz |
| PE14 | E14 | SPI4_MOSI | EMVS_ADC_SDI | OUT | AD7380-4 #1/#2 SDI | 3.3 | Config writes |
| PE13 | E13 | SPI4_MISO | EMVS_ADC_SDOA | IN | AD7380-4 active SDOA | 3.3 | Ch A data; muxed via CS |
| PE5 | E5 | GPIO | EMVS_ADC_SDOB | IN | AD7380-4 active SDOB | 3.3 | Ch B data; GPIO-capture |
| PE6 | E6 | GPIO | EMVS_ADC_SDOC | IN | AD7380-4 active SDOC | 3.3 | Ch C data; GPIO-capture |
| PE7 | E7 | GPIO | EMVS_ADC_SDOD | IN | AD7380-4 active SDOD | 3.3 | Ch D data; GPIO-capture |
| PF4 | F4 | GPIO | CS_AD7380_1 | OUT | AD7380-4 #1 CS | 3.3 | Active low; also triggers conversion |
| PF5 | F5 | GPIO | CS_AD7380_2 | OUT | AD7380-4 #2 CS | 3.3 | Active low; also triggers conversion |

Rev D frees 16 STM32 analog input pins (8 differential pairs) previously used for on-die EMVS ADC sampling. STM32 internal ADC1+ADC2 are now available for housekeeping, temperature monitoring, or future sensor integration.

## 7.8  Power & Enable Control

| STM32 Pin | Port | Alt Fn | Signal Name | Dir | Destination | V | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PE0 | E0 | GPIO | TX_EN | OUT | LDO EN: HMC1131 ×2, ADF5901 ×2 | 3.3 | TX chain master enable |
| PE1 | E1 | GPIO | RX_EN | OUT | LDO EN: PMA3 ×2, ADF5904 #1 | 3.3 | Main-RX enable |
| PE2 | E2 | GPIO | EMVS_EN | OUT | LDO EN: ADAR2001, ADAR2004 ×2, AD7380-4 ×2, LMX2594 | 3.3 | EMVS chain |
| PE3 | E3 | GPIO | TCXO_EN | OUT | 100 MHz TCXO EN | 3.3 | Enable first; 10 ms settle |
| PE4 | E4 | GPIO | PLL_EN | OUT | ADF4159 CE ×2, ADF5904 #1 CE | 3.3 | After TCXO stable |

## 7.9  Total STM32 pin usage summary

| Category | Count | Notes |
| --- | --- | --- |
| SPI6 config bus | 3 | PA5/PA6/PA7 |
| SPI4/5 + AD7380-4 ×3 | 9 | SCK, MOSI, MISO(SDOA), SDOB, SDOC, SDOD, CS×3 |
| SPI chip selects (GPIO) | 9 | 5× ADF + 2× ADAR2004 + LMX2594 (ADAR2001 CS via MAX7301) |
| Chirp timing (TIM1) | 2 | CNV_MASTER + CHIRP_START |
| TX/BPSK control | 6 | 2× TXDATA + 2× BPSK + 2× LD_ADF4159 |
| LMX2594 lock detect | 1 | PF3 |
| Master enables | 5 | TX, RX, EMVS, TCXO, PLL |
| HSE clock | 2 | STM32-local XTAL |
| TOTAL | 37 | Of ~165 usable GPIO on VFBGA264 (down from 43 with AD4630) |

Rev E GPIO allocation: Pin 50 (PA10, was RxRST) repurposed as ADAR2001 TxADV for per-chirp pol cycling. ADAR2001 CS and RxRST function moved to MAX7301 I/O expander (P20, P21). MADV/MRST shared. SPI6 (RF_SPI) bus for all RF config. All SP4T and internal ADC pins from Rev C fully eliminated.

# 8.  Per-Component Pin Assignments

Pin number, datasheet name, signal type, connection, and notes for each component. Pin numbers per manufacturer datasheet.

## 8.  RF Component Pin Tables
Note: All SPI and CS connections in §8.1–§8.10 now reference the B2B connector pins and MAX7301 I/O expander ports on the analog board, per the connector ICD in §13. ADF4159/ADF5901/ADF5904/BSS138 operate at 3.3 V SPI (no level shifting). ADAR2001/ADAR2004/LMX2594 operate at 1.8 V SPI (TXS0108E level shifting on analog board). AD7380-4 ADCs are on the digital board and use direct STM32 SPI4 pins.
8.1  ADF4159Chirp PLL  (×2, LFCSP-24 4×4 mm)

Both ADF4159 #1 and #2 identical pinout; nets differ by _1 / _2 suffix.

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | RSET | Analog | 10 kΩ to AGND | ICP = 25.5/RSET; standard 10 k = 2.55 mA |
| 2 | AVDD | Power | 3.3 V analog | 100 nF + 10 µF |
| 3 | CPGND | GND | Analog GND |  |
| 4 | CPOUT | Analog | Loop filter → ADF5901 #N VTUNE | 2nd/3rd-order active LF |
| 5 | DVDD | Power | 1.8 V digital |  |
| 6 | DGND | GND | Digital GND |  |
| 7 | AGND | GND | Analog GND |  |
| 8 | RFIN A | RF IN | ADF5901 #N LO_OUT (10 pF AC-couple) | 13 GHz VCO feedback |
| 9 | RFIN B | RF IN | 50 Ω term | Single-ended input |
| 10 | AGND | GND | Analog GND |  |
| 11 | REFIN | CLK IN | 100 MHz TCXO | AC-couple 100 pF |
| 12 | DVDD1 | Power | 1.8 V digital |  |
| 13 | TXDATA | Digital | Connector pin 38/PN12 (#1) / pin 40/PA4 (#2) | 3.3 V LVTTL |
| 14 | DATA | Digital | RF_SPI_MOSI (connector pin 32) | Shared |
| 15 | CLK | Digital | RF_SPI_SCK (connector pin 30) | Shared |
| 16 | LE | Digital | MAX7301 P4 (#1) / P5 (#2) | Active low |
| 17 | CE | Digital | MAX7301 P16 (PLL_EN) | Pull-up 10k |
| 18 | MUXOUT | Digital | MAX7301 P17 (#1) / P18 (#2) | Config as Lock-Detect |
| 19 | SW | Analog | NC / GND via 100 nF | Unused FMCW |
| 20 | V_P | Power | 5.0 V via LDO | CP supply; ≥ AVDD+0.3 |
| 21 | AVDD_VCO | Power | 3.3 V |  |
| 22 | AVDD_VCO | Power | 3.3 V |  |
| 23 | AGND | GND | Analog GND |  |
| 24 | (see DS) | — | Verify ADF4159 DS Rev E | Some revs renumber |
| EPAD | EPAD | GND | Thermal via array |  |

## 8.2  ADF590124 GHz TX VCO+PA  (×2, LFCSP-32 5×5 mm)

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | DGND | GND | Digital GND |  |
| 2 | DVDD | Power | 1.8 V digital |  |
| 3 | DATA | Digital | RF_SPI_MOSI (connector pin 32) |  |
| 4 | CLK | Digital | RF_SPI_SCK (connector pin 30) |  |
| 5 | LE | Digital | MAX7301 P6 (#1) / P7 (#2) | Active low |
| 6 | CE | Digital | MAX7301 P12 (TX_EN) |  |
| 7 | REFIN | CLK IN | 100 MHz TCXO | AC-couple |
| 8 | AGND | GND | Analog GND |  |
| 9 | VTUNE | Analog | ADF4159 #N CPOUT via LF | 0.5–4.5 V |
| 10 | AVDD_VCO | Power | 3.3 V |  |
| 11 | TX1_OUT | RF OUT | HMC1131 #N RFIN | +8 dBm |
| 12 | TX2_OUT | RF OUT | 50 Ω term | Unused |
| 13 | AGND | GND |  |  |
| 14 | LO_OUT | RF OUT | #1 → ADF5904 #1 LO_IN / #2 → 50 Ω term | -1 dBm |
| 15–20 | AVDD_x | Power | 3.3 V per DS |  |
| 21–28 | AGND / int | GND / A | Per DS |  |
| 29 | TX_AMP_EN1 | Digital | Int bias via R to CE | Per-ch PA EN via SPI |
| 30 | TX_AMP_EN2 | Digital | 50 Ω or GND | TX2 unused |
| 31 | AVDD | Power | 3.3 V |  |
| 32 | DGND | GND |  |  |
| EPAD | EPAD | GND | Thermal via array | 170 mA dissipation |

## 8.3  HMC1131LC4TR24–35 GHz MPA  (×2, LCC-24)

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | RFIN | RF IN | ADF5901 #N TX1_OUT | +8 dBm |
| 2 | GND | GND |  |  |
| 3 | VD1 | Power | 5.0 V (Rbias) | ~40 mA |
| 4 | VG1 | Power | −0.5 to −1.5 V |  |
| 5–7 | GND | GND |  |  |
| 8 | VD2 | Power | 5.0 V | ~75 mA |
| 9 | VG2 | Power | Gate bias |  |
| 10–12 | GND | GND |  |  |
| 13 | VD3 | Power | 5.0 V | ~110 mA |
| 14 | VG3 | Power | Gate bias |  |
| 15–23 | GND | GND |  |  |
| 24 | RFOUT | RF OUT | TX1 K-band patch feed (Rev E: TX-EMVS 1 on separate ADAR2001 path) | +30 dBm |
| EPAD | EPAD | GND | Thermal CRITICAL | 1.125 W dissipation |

## 8.4  PMA3-24323LN+24–32 GHz LNA  (×2)

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | RF IN | RF IN | RX SA2 (#1) / RX SA3 (#2) | 50 Ω |
| 2 | GND | GND |  |  |
| 3 | RF OUT | RF OUT | ADF5904 #1 RFIN_A+ / RFIN_B+ | 50 Ω matched |
| 4 | VCC | Power | 3.3 V via bias tee | ~30 mA |
| EPAD | EPAD | GND |  |  |

## 8.5  ADF5904 #14-Channel Receiver  (LFCSP-32 5×5 mm)

Rev D: Ch A/B used for main beams. Ch C/D inputs now OPEN (Ch A/B → CH9/CH10 → ADC3. Ch C/D unused.). Ch C/D IF outputs available for future use.

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | RFIN_A+ | RF IN | PMA3 #1 OUT (Beam 1) | Diff pair |
| 2 | RFIN_A- | RF IN |  |  |
| 3 | RFIN_B+ | RF IN | PMA3 #2 OUT (Beam 2) |  |
| 4 | RFIN_B- | RF IN |  |  |
| 5 | RFIN_C+ | RF IN | 50 Ohm term (FREE) | Rev D: disconnected from EMVS |
| 6 | RFIN_C- | RF IN | 50 Ohm term (FREE) |  |
| 7 | RFIN_D+ | RF IN | 50 Ohm term (FREE) | Rev D: disconnected from EMVS |
| 8 | RFIN_D- | RF IN | 50 Ohm term (FREE) |  |
| 9 | LO_IN | RF IN | ADF5901 #1 LO_OUT | -1 dBm |
| 10 | AGND | GND |  |  |
| 11 | IF_A+ | IF OUT | Connector CH9P (pin 51) → AD7380-4 #3 AINA+ (Rev E remap) | Diff, 0-2 MHz beat freq |
| 12 | IF_A- | IF OUT | Connector CH9N (pin 53) → AD7380-4 #3 AINA- |  |
| 13 | IF_B+ | IF OUT | Connector CH10P (pin 57) → AD7380-4 #3 AINB+ (Rev E remap) |  |
| 14 | IF_B- | IF OUT | Connector CH10N (pin 59) → AD7380-4 #3 AINB- |  |
| 15-22 | IF_C/D | IF OUT | OPEN / future use | Rev D: freed by ADAR2004 swap |
| 23 | DATA | Digital | RF_SPI_MOSI (connector pin 32) |  |
| 24 | CLK | Digital | RF_SPI_SCK (connector pin 30) |  |
| 25 | LE | Digital | MAX7301 P8 (CS_ADF5904) | Active low |
| 26 | CE | Digital | MAX7301 P13 (RX_EN) |  |
| 27 | MUXOUT | Digital | RF_SPI_MISO (connector pin 34) | Tri-state on CS-deassert |
| 28 | REFIN | CLK IN | 100 MHz TCXO | LO buffer cal |
| 29-32 | DVDD/AVDD | Power | 1.8 V / 3.3 V |  |
| EPAD | EPAD | GND | Thermal via array |  |

## 8.6  ADAR2004ACCZ4-Ch Rx Mixer  (x2, LGA-48 7x7 mm)

Integrated 4-channel receiver with LNA + mixer per channel, on-die x4 LO multiplier, SPI-controlled sequencer. 10-40 GHz RF, 2.5 V supply, 910 mW total.

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1-4 | RF1-RF4 | RF IN | EMVS Cell #N probes (Ex, Ey, Hx, Hy) | 50 Ohm, 10-40 GHz |
| 5-8 | IF1-IF4 | IF OUT | AD7380-4 #N (AIN_A+, AIN_B+, AIN_C+, AIN_D+) | Differential, 0-2 MHz |
| 9 | LO_IN | RF IN | LMX2594 via Wilkinson splitter | +3.5 dBm, 6 GHz |
| 10 | VCC | Power | 2.5 V via EMVS_EN LDO | 910 mW total |
| 11 | CSB | Digital | MAX7301 P9 (#1) / P10 (#2) via TXS0108E | Active low; SPI6 (RF_SPI) bus |
| 12 | SCLK | Digital | RF_SPI_SCK (connector pin 30) | Shared |
| 13 | SDI | Digital | RF_SPI_MOSI (connector pin 32) | Shared |
| 14 | SDO | Digital | RF_SPI_MISO (connector pin 34) | Tri-state |
| 15-48 | GND/NC | GND | Per datasheet | Thermal via array on EPAD |

## 8.7  AD7380-4BCPZQuad 16-bit SAR ADC  (x2, LFCSP-24 4x4 mm)

4-channel simultaneous sampling, 4 MSPS, differential inputs, external 2.5 V reference. Native 3.3 V SPIno level shifter needed.

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1-8 | AIN_A-D +/- | Analog | IF inputs from connector (Cell 1: CH5-8, Cell 2: CH1-4, Main: CH9-10) | Diff pairs, Vcm = VCC/2 |
| 9 | REFIN | Analog | ADR4525 2.5 V ref (shared) | 1 uF decoupling |
| 10 | VCC | Power | 3.3 V via EMVS_EN LDO | ~30 mW per device |
| 11 | VLOGIC | Power | 3.3 V | I/O level; 1.8-3.3 V compatible |
| 12 | GND | GND |  |  |
| 13 | CS | Digital | STM32 PF6 (#1=Cell2) / PF4 (#2=Cell1) / PF5 (#3=Main) | Active low; initiates conversion |
| 14 | SCLK | Digital | STM32 PE12 SPI4_SCK (on digital board) | Shared |
| 15 | SDI | Digital | STM32 PE14 SPI4_MOSI (on digital board) | Register config |
| 16 | SDOA | Digital | STM32 PE13 SPI4_MISO (on digital board) | Ch A data |
| 17 | SDOB | Digital | STM32 PE5 GPIO (on digital board) | Ch B data |
| 18 | SDOC | Digital | STM32 PE6 GPIO (on digital board) | Ch C data |
| 19 | SDOD | Digital | STM32 PE7 GPIO (on digital board) | Ch D data / ALERT |
| 20-24 | GND/NC | GND |  |  |
| EPAD | EPAD | GND | Connect to GND |  |

## 8.8  LMX2594RHAR6 GHz PLL+VCO  (VQFN-40 6x6 mm)

Dedicated EMVS LO synthesizer. Generates 6 GHz from 100 MHz TCXO reference; ADAR2004 on-die x4 multiplier produces 24 GHz internal LO. VCO runs at 12 GHz with /2 output divider.

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | OSCin+ | CLK IN | 100 MHz TCXO (AC-coupled) | Shared reference bus |
| 2 | OSCin- | CLK IN | GND via 100 pF (single-ended mode) |  |
| 3-6 | VCC_x | Power | 3.3 V (integrated LDOs) | 100 nF + 10 uF per pin |
| 7 | MUXout | Digital | MAX7301 P19 (lock detect) via TXS0108E | Lock detect |
| 8 | CSB | Digital | MAX7301 P11 (CS_LMX2594) via TXS0108E | Active low; SPI6 (RF_SPI) bus |
| 9 | SCK | Digital | RF_SPI_SCK (connector pin 30) | Shared |
| 10 | SDI | Digital | RF_SPI_MOSI (connector pin 32) | Shared |
| 11 | RFoutA+ | RF OUT | RFoutA → 2-way Wilkinson → ADAR2004 #1 + #2 | +7 dBm @ 6 GHz |
| 12 | RFoutA- | RF OUT | RFoutB → ADAR2001 RFIN direct (+7 dBm). Rev E: was 50 ohm term. |  |
| 13-40 | GND/NC/VCC | Power/GND | Per datasheet |  |
| EPAD | EPAD | GND | Thermal via array | ~450 mW |

## 8.9  BSS138 — BPSK Loop-Filter Gate  (x2, SOT-23)

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 (G) | Gate | Digital | Connector pin 42/PG11 (#1) / pin 44/PG2 (#2) | HIGH = short bypass C |
| 2 (S) | Source | GND | System GND |  |
| 3 (D) | Drain | Analog | ADF4159 #N loop-filter node across Cbypass | Shorts C when HIGH |

## 8.10  100 MHz TCXO

| Pin # | Pin Name | Type | Connection | Notes |
| --- | --- | --- | --- | --- |
| 1 | EN | Digital | MAX7301 P15 (TCXO_EN) | HIGH enabled |
| 2 | GND | GND |  |  |
| 3 | OUT | CLK OUT | Fanout -> ADF4159 x2, ADF5901 x2, ADF5904, LMX2594 REFIN | 1-to-6 buffer; skew <50 ps |
| 4 | VCC | Power | 3.3 V always-on |  |

# 9.  Net Naming Convention

| Pattern | Meaning | Example |
| --- | --- | --- |
| <FUNCTION>_<INDEX> | Per-instance control nets | CS_ADF4159_1, TXDATA_2 |
| <CHIP>_<PIN> | Chip-to-chip direct | ADF5901_1_LO_OUT, ADAR2004_1_IF2_P |
| <BUS>_<SIGNAL> | Shared buses | SPI_SCK, RADAR_REF_100MHZ |
| <DOMAIN>_<VOLTAGE> | Power rails | VCC_3V3, VCC_5V_TX, VCC_1V8_DIG |
| EMVS<N>_<BAND>_<IQ>± | EMVS differential IF pairs | EMVS1_K_I+, EMVS2_X_Q− |
| <TEST>_<FUNCTION> | Test/scope points | TP_CNV, TP_CHIRP_START |

# 10.  Power Domain Summary

| Rail | Voltage | Sourced by | Load | Est. Current | Enable |
| --- | --- | --- | --- | --- | --- |
| VCC_3V3_ALWAYS | 3.3 V | Always-on LDO | TCXO, STM32, AD7380-4 #1 (EMVS Cell 2 ADC) | ~90 mA | (always) |
| VCC_5V_TX | 5.0 V | TX_EN LDO | HMC1131 x2 drains | ~450 mA | TX_EN |
| VCC_3V3_TX | 3.3 V | TX_EN LDO | ADF5901 x2 AVDD | ~340 mA | TX_EN |
| VCC_3V3_RX | 3.3 V | RX_EN LDO | PMA3 x2, ADF5904 #1 AVDD | ~120 mA | RX_EN |
| VCC_2V5_EMVS | 2.5 V | EMVS_EN LDO | ADAR2001 (450 mW) + ADAR2004 x2 (910 mW each) | ~910 mA | EMVS_EN |
| VCC_3V3_EMVS | 3.3 V | EMVS_EN LDO | AD7380-4 x2 (EMVS), LMX2594, ADR4525 ref | ~200 mA | EMVS_EN |
| VCC_5V_CP | 5.0 V | PLL_EN LDO | ADF4159 V_P (charge-pump) | ~10 mA | PLL_EN |
| TOTAL |  |  |  | ~2.1 A peak |  |

Rev E notes: ADAR2001 adds 450 mW (180 mA) to EMVS 2.5 V rail. Total EMVS dissipation now 2.27 W (ADAR2001 450 mW + ADAR2004 x2 1.82 W). Thermal via array critical under all three LGA packages.

# 11.  STM32 Peripheral Allocation Summary

| Peripheral | Function | Channels / Pins | Notes |
| --- | --- | --- | --- |
| SPI6 | RF config bus (shared) | SPI6 via connector pins 30/32/34 + MAX7301 CS (pin 26) | ADF4159 x2, ADF5901 x2, ADF5904, ADAR2001, ADAR2004 x2, LMX2594 |
| SPI4/5 | AD7380-4 x3 unified ADC bus | SCK/MOSI/MISO + 3 GPIO CS + 3 SDO aux | Main beams + EMVS x2, all native 3.3 V |
| TIM1_CH1 | Master CNV pulse | PE9 | 2 MHz, AD7380-4 #1 CNV + AD7380-4 sync |
| TIM1_CH2 | Chirp-start trigger | PE11 | 28.6 kHz |
| GPDMA Ch 1 | AD7380-4 #1 SDO_B capture | PB4 -> memory |  |
| GPDMA Ch 2 | AD7380-4 #1 SDO_A capture | SPI3_MISO -> memory |  |
| GPDMA Ch 4 | AD7380-4 SPI4 capture | SPI4 -> memory | 4-wire simultaneous readout |
| ADC1+ADC2 | FREED (Rev D) | All internal ADC pins available | Housekeeping / temperature / future |
| GPIO Bank B | SPI CS (ADF4159/5901) | PB12-PB15 |  |
| GPIO Bank D | TXDATA, BPSK, LD, CS (ADF5904) | PD0-PD5, PD8 |  |
| GPIO Bank E | Master EN, TIM, SPI4 | PE0-PE4, PE9, PE11-PE14 |  |
| GPIO Bank F | ADAR2004 CS, LMX2594 CS/LD, AD7380 CS | PF0-PF5 |  |
| HSE | STM32 core clock | PH0/PH1 (25 MHz XTAL) | Independent of radar TCXO |
| Ethos-U65 | Polarimetric classifier | (internal) | No external pins |

# 12.  Open Items and Validation Checklist

Items CLOSED by Rev D:

- CLOSED: ADF5904 IQ architecture question -- Ch C/D no longer used for EMVS. ADAR2004 provides dedicated downconversion.

- CLOSED: ADC simultaneity problem -- AD7380-4 provides 4 true-simultaneous channels per cell natively.

- UPDATED (Rev E): TXS0108E level shifters required on analog board for ADAR2001/ADAR2004/LMX2594 (1.8 V I/O). ADC path (AD7380-4) remains native 3.3 V — no level translation on ADC SPI bus.

- CLOSED: ADRF5024 VSS polarity question -- part removed from design.

- CLOSED: PA4/PA5 SPI1 conflict -- internal ADC pins freed entirely.

- CLOSED: AD4630-24 removed -- replaced by AD7380-4 #1 on shared SPI bus. Eliminates SPI3, TXS0108E, 1.8V digital rail, and 5V ADC rail. Main beam resolution drops 24-bit to 16-bit (adequate for FMCW radar).

Items CLOSED by Rev E:

- CLOSED: X-band TX gap -- ADAR2001 drives TX-EMVS 1 cell with full polarimetric TX diversity (Ex/Ey/Hx/Hy). LMX2594 RFoutB (was terminated in Rev D) now feeds ADAR2001 directly; RFoutA unchanged (2-way Wilkinson to ADAR2004 x2). No 3-way splitter needed.

- CLOSED: ADAR2001 control with fixed digital board -- CS via MAX7301 I/O expander P20 (analog board, SPI6 bus); MADV/MRST shared on RF board; pin 50 (RxRST) repurposed as TxADV; RxRST moved to MAX7301 P21. Zero digital board changes. Analog board connector ICD added §13.

Items REMAINING or NEW in Rev E:

- STM32N657 SPI4/SPI5 AF pin verification -- PE12/PE13/PE14 assumed as SPI4 from H7 family patterns. Nucleo-N657X0-Q Zephyr DTS shows PE13/PE14 as I2C4 and PE15 as SPI5. Verify in CubeMX or datasheet Table 19. Fallback: use SPI5 on PE15/PG1/PG2.

- STM32N657 package correction -- device is VFBGA264 (14x14 mm, 0.8 mm pitch), NOT LQFP144. Pin assignments use GPIO port names (correct) but physical ball coordinates need mapping from BGA ballout diagram.

- ADAR2004 control pins -- verify PDN (power-down) and RESET pin requirements from datasheet. May need dedicated GPIO or tie to EMVS_EN rail.

- LMX2594 CE pin -- currently unassigned. Recommend tying to VCC (always enabled) to avoid PLL lock-up delay on EMVS enable.

- CLOSED (Rev E): Bow-tie antennas eliminated. TX-EMVS 1 cell replaces both bow-ties. No HFSS re-tune needed — EMVS cell is broadband by design.

- ADAR2004 thermal plan -- 910 mW x 2 = 1.82 W on two LGA-48 packages. Via array and copper pour sizing TBD.

- LMX2594 loop filter design for 6 GHz output. TI TICS Pro tool.

- AD7380-4 external reference -- ADR4525 (2.5 V) shared across all 3 AD7380-4 devices via star distribution.

- LO splitter drive budget at 6 GHz -- verify ADAR2004 minimum LO drive spec vs +3.5 dBm available.

- ADF5904-to-AD7380-4 DC bias matching -- ADF5904 IF common-mode is ~VDD/2 (1.65 V). Verify AD7380-4 input common-mode range accepts this directly. May need resistive divider or AC coupling caps if mismatch.

# 13.  Analog Board Mating Connector — Pin Allocation & Signal Routing

Connector: Molex 204928-0601 (plug, on digital board) mating with Molex 205266-0601 (receptacle, on analog board). 60-pin, 0.4 mm pitch, 0.6 mm stack height. Pin 1 on plug mates with pin 1 on receptacle (1:1 mapping). 2 rows of 30 pins: odd row carries differential analog pairs with GND shields, even row carries power, SPI buses, and control GPIOs.

## 13.1  Power Pins

VBATT (pins 4, 6, 8, 10) — Raw 1S LiPo, 3.0–4.2 V. Input to analog board LDOs for 3.3 V and 2.5 V generation. Tie all four pins to common VBATT rail. Budget ≤500 mA continuous. CRITICAL: At end-of-discharge (3.0 V), standard 3.3 V LDOs cannot regulate. Use ultra-low-dropout LDO (e.g., TLV75533, 55 mV dropout) or buck-boost converter (e.g., TPS63001) for the analog board 3.3 V rail. The 2.5 V EMVS rail (ADAR2001/2004) has adequate headroom from 3.0 V input.

V1P8 (pins 14, 16) — Regulated 1.8 V from digital board LP5912 LDO. Budget ≤200 mA for analog board. Use for low-noise analog references only, not ADAR2004/2001 digital supply.

GND (pins 1, 2, 7, 12, 13, 19, 25, 28, 31, 36, 37, 43, 48, 49, 55, 60, MP1–MP4) — 14 signal GND plus 4 mechanical pegs. Odd-pin GNDs are shields between differential ADC pairs; maintain GND plane continuity on analog board under connector footprint.

## 13.2  RF Configuration Bus — SPI6 (RF_SPI)

| Pin | Signal | STM32 | Dir | Analog Board Destination |
| --- | --- | --- | --- | --- |
| 30 | RF_SPI_SCK | SPI6_SCK (V10) | In | MAX7301 SCK + all RF chip SCLK (shared bus) |
| 32 | RF_SPI_MOSI | SPI6_MOSI (T7) | In | MAX7301 DIN + all RF chip SDI (shared bus) |
| 34 | RF_SPI_MISO | SPI6_MISO (W15) | Out | MAX7301 DOUT + RF chip SDO (tri-state, shared) |
| 26 | CS_IO_EXP | PA11 GPIO (T8) | In | MAX7301 CS (I/O expander). Only CS across connector. |

IMPORTANT: All SPI6 signals (SCK, MOSI, MISO) and CS_IO_EXP cross the connector at 3.3 V CMOS but ADAR2001, ADAR2004, and LMX2594 require 1.8 V digital I/O. The analog board must include bidirectional level shifters (TXS0108E or equivalent) between the connector pads and the 1.8 V domain. VCCA (1.8 V) is sourced from the analog board EMVS 1.8 V LDO; VCCB (3.3 V) is sourced from the analog board 3.3 V rail. The MAX7301 operates at 3.3 V (no level shifting needed for the expander itself). ADF4159, ADF5901, and ADF5904 also operate at 3.3 V SPI — no level shifting needed for those chips. Only ADAR2001, ADAR2004 ×2, and LMX2594 require 1.8 V translation. All RF chip-select lines are generated locally on the analog board by the MAX7301 I/O expander. Only CS_IO_EXP crosses the connector. SPI transaction sequence: assert CS_IO_EXP → write MAX7301 to set target CS low → deassert CS_IO_EXP → clock RF data → reassert CS_IO_EXP → write MAX7301 to release CS.

## 13.3  General-Purpose Expansion Bus — SPI3 (EXP_SPI)

| Pin | Signal | STM32 | Dir | Analog Board Destination |
| --- | --- | --- | --- | --- |
| 18 | EXP_SPI_SCK | PC10 SPI3_SCK | In | Expansion peripheral CLK |
| 20 | EXP_SPI_MISO | PC11 SPI3_MISO | Out | Expansion peripheral DOUT |
| 22 | EXP_SPI_MOSI | PC12 SPI3_MOSI | In | Expansion peripheral DIN |
| 24 | EXP_SPI_NCS | PA15 SPI3_NSS | In | Expansion peripheral CS (active low) |

Independent of RF_SPI (SPI6). Available for future expansion peripherals. 10 kΩ pull-up on NCS to 3.3 V recommended on analog board.

## 13.4  Per-Chirp RF Control GPIOs (Direct STM32 — Timing Critical)

These signals are toggled per-chirp by firmware and require direct STM32 GPIO for sub-microsecond timing. They bypass the MAX7301 I/O expander. All cross the connector at 3.3 V CMOS. LEVEL SHIFTING REQUIRED: MRST (pin 46), TxADV (pin 50), RxADV (pin 54), and MADV (pin 58) drive ADAR2001/ADAR2004 inputs at 1.8 V — route through TXS0108E 3.3 V → 1.8 V translation on the analog board. TXDATA_1/2 and BPSK_GATE_1/2 drive ADF4159/BSS138 at 3.3 V — no level shifting needed for those four signals.

| Pin | Signal | STM32 | Analog Board Destination | Notes |
| --- | --- | --- | --- | --- |
| 38 | TXDATA_1 | PN12 GPIO | ADF4159 #1 TXDATA | DDM-MIMO π-phase code, toggles every chirp |
| 40 | TXDATA_2 | PA4 GPIO | ADF4159 #2 TXDATA | DDM-MIMO π-phase code, toggles every chirp |
| 42 | BPSK_GATE_1 | PG11 GPIO | BSS138 #1 gate (ADF4159 #1 LF) | BPSK loop-filter gate drive |
| 44 | BPSK_GATE_2 | PG2 GPIO | BSS138 #2 gate (ADF4159 #2 LF) | BPSK loop-filter gate drive |
| 46 | MRST | PB12 GPIO | ADAR2001 + ADAR2004 #1 + #2 MRST (tied) | Multiplier reset, 3-way shared on analog board |
| 50 | TxADV | PA10 GPIO | ADAR2001 TxADV only | Rev E: was RxRST. Per-chirp pol cycling (Ex/Ey/Hx/Hy) |
| 54 | RxADV | PG15 GPIO | ADAR2004 #1 + #2 RxADV (tied) | Receiver gain/channel state advance |
| 58 | MADV | PG13 GPIO | ADAR2001 + ADAR2004 #1 + #2 MADV (tied) | Multiplier advance, 3-way shared on analog board |

## 13.5  Hardware-Paced Radar Timing

| Pin | Signal | STM32 Timer | Rate | Analog Board Use |
| --- | --- | --- | --- | --- |
| 52 | CNV_MASTER | TIM2_CH2 (W12) | 2 MHz | AD7380-4 ×3 CNV (on digital board; sync ref on analog board if needed) |
| 56 | CHIRP_START | TIM2_CH3 (W10) | 28.6 kHz | Hardware chirp-rate reference pulse (28.6 kHz). Sync for LMX2594 ramp, TxADV timing, scope trigger. |

Both signals are TIM2 channels with microsecond-precise phase alignment. 3.3 V CMOS output from digital board.

## 13.6  Differential Analog Inputs (ADF_CH1–CH10)

10 differential pairs carrying IF outputs from the analog board to AD7380-4 SAR ADCs on the digital board. Output from analog board. Full-scale: ±2.5 V differential, common-mode 1.25 V (VREF/2 from LTC6655B-2.5 on digital board). 33 Ω + 68 pF anti-alias filter on digital board at ADC pin. IMPORTANT: When EMVS_EN (P14) is deasserted, ADAR2004 IF outputs go high-impedance. Add 100 kΩ bias resistors to VREF/2 (1.25 V) on each differential leg on the analog board to prevent ADC input latch-up. Alternatively, firmware must halt ADC1/ADC2 sampling before deasserting EMVS_EN.

| Pins (+/−) | Signal | Digital Board ADC | ADC Ch | Analog Board IF Source |
| --- | --- | --- | --- | --- |
| 3 / 5 | CH1 | ADC1 (AD7380-4 #1) | AINA | ADAR2004 #2 IF Ch1 (RX-EMVS 2 Ex) |
| 9 / 11 | CH2 | ADC1 | AINB | ADAR2004 #2 IF Ch2 (RX-EMVS 2 Ey) |
| 15 / 17 | CH3 | ADC1 | AINC | ADAR2004 #2 IF Ch3 (RX-EMVS 2 Hx) |
| 21 / 23 | CH4 | ADC1 | AIND | ADAR2004 #2 IF Ch4 (RX-EMVS 2 Hy) |
| 27 / 29 | CH5 | ADC2 (AD7380-4 #2) | AINA | ADAR2004 #1 IF Ch1 (RX-EMVS 1 Ex) |
| 33 / 35 | CH6 | ADC2 | AINB | ADAR2004 #1 IF Ch2 (RX-EMVS 1 Ey) |
| 39 / 41 | CH7 | ADC2 | AINC | ADAR2004 #1 IF Ch3 (RX-EMVS 1 Hx) |
| 45 / 47 | CH8 | ADC2 | AIND | ADAR2004 #1 IF Ch4 (RX-EMVS 1 Hy) |
| 51 / 53 | CH9 | ADC3 (AD7380-4 #3) | AINA | ADF5904 Ch A IF (Main RX Beam 1) |
| 57 / 59 | CH10 | ADC3 | AINB | ADF5904 Ch B IF (Main RX Beam 2) |

Rev E analog board remap: ADC1 (CH1-CH4) = EMVS Cell 2 (ADAR2004 #2, all 4 channels: Ex/Ey/Hx/Hy). ADC2 (CH5-CH8) = EMVS Cell 1 (ADAR2004 #1, all 4 channels). ADC3 (CH9-CH10) = Main RX beam (ADF5904 Ch A/B). Each ADC serves exactly one signal source — no mixed DMA buffers. ADF5904 Ch C/D are not connected to the expansion connector. The swap is purely analog board trace routing; no digital board or connector changes.

## 13.7  MAX7301 I/O Expander Port Assignment (Analog Board)

MAX7301AAX+ (Analog Devices, 28-port SPI GPIO expander). Powered from 3.3 V on analog board. SPI Mode 0, ≤26 MHz. Controlled via SPI6 with CS_IO_EXP (pin 26/PA11). All ports default to input (high-Z) at power-on. Active-low CS lines have 10 kΩ pull-up; active-high enable lines have 10 kΩ pull-down for safe boot state.

| Port | Signal | Dir | Active | Drives / Reads | Pull |
| --- | --- | --- | --- | --- | --- |
| P4 | CS_ADF4159_1 | Out | Low | ADF4159 #1 LE | 10k up |
| P5 | CS_ADF4159_2 | Out | Low | ADF4159 #2 LE | 10k up |
| P6 | CS_ADF5901_1 | Out | Low | ADF5901 #1 LE | 10k up |
| P7 | CS_ADF5901_2 | Out | Low | ADF5901 #2 LE | 10k up |
| P8 | CS_ADF5904_1 | Out | Low | ADF5904 #1 LE | 10k up |
| P9 | CS_ADAR2004_1 | Out | Low | ADAR2004 #1 CSB | 10k up |
| P10 | CS_ADAR2004_2 | Out | Low | ADAR2004 #2 CSB | 10k up |
| P11 | CS_LMX2594 | Out | Low | LMX2594 CSB | 10k up |
| P12 | TX_EN | Out | High | TX-chain LDO (HMC1131 ×2, ADF5901 ×2) | 10k dn |
| P13 | RX_EN | Out | High | Main-RX LDO (PMA3 ×2, ADF5904) | 10k dn |
| P14 | EMVS_EN | Out | High | EMVS LDO (ADAR2001, ADAR2004 ×2, LMX2594) | 10k dn |
| P15 | TCXO_EN | Out | High | 100 MHz TCXO enable | 10k dn |
| P16 | PLL_EN | Out | High | ADF4159 CE ×2, ADF5904 CE | 10k dn |
| P17 | LD_ADF4159_1 | In | — | ADF4159 #1 lock detect (low = loss) | 56k up |
| P18 | LD_ADF4159_2 | In | — | ADF4159 #2 lock detect (low = loss) | 56k up |
| P19 | LD_LMX2594 | In | — | LMX2594 lock detect (low = loss) | 56k up |
| P20 | CS_ADAR2001 | Out | Low | Rev E: ADAR2001 CSB | 10k up |
| P21 | RxRST | Out | High | Rev E: ADAR2004 Rx reset (relocated from pin 50) | — |
| P22 | TxRST | Out | High | Rev E: ADAR2001 TX reset | — |
| P23 | ADAR2001_TXEN | — | — | P23: ADAR2001_TXEN (Out, High, 10k pull-dn) — PA enable. P24: ADAR2001_FAULT (In, 56k pull-up) — over-temp/fault flag. P25-P31: Reserved spare. | — |

## 13.8  Complete 60-Pin Connector Table — Digital Board vs Analog Board

Master cross-reference: every pin with its STM32 source (digital board) and component destination (analog board). Direction is from the analog board perspective: In = signal driven by digital board, Out = signal driven by analog board.

| Pin | Signal | STM32 Ball / Function | Dir | Analog Board Destination | Group |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | — | — | GND shield CH1 | — |
| 2 | GND | — | — | GND plane | — |
| 3 | ADF_CH1P | ADC1 AINA+ | Out | ADAR2004 #2 IF Ch1+ (RX-EMVS 2 Ex) | Analog |
| 4 | VBATT | 1S LiPo rail | In | Analog board LDO input | Power |
| 5 | ADF_CH1N | ADC1 AINA− | Out | ADAR2004 #2 IF Ch1− (RX-EMVS 2 Ex) | Analog |
| 6 | VBATT | 1S LiPo rail | In | Analog board LDO input | Power |
| 7 | GND | — | — | GND shield CH1/CH2 | — |
| 8 | VBATT | 1S LiPo rail | In | Analog board LDO input | Power |
| 9 | ADF_CH2P | ADC1 AINB+ | Out | ADAR2004 #2 IF Ch2+ (RX-EMVS 2 Ey) | Analog |
| 10 | VBATT | 1S LiPo rail | In | Analog board LDO input | Power |
| 11 | ADF_CH2N | ADC1 AINB− | Out | ADAR2004 #2 IF Ch2− (RX-EMVS 2 Ey) | Analog |
| 12 | GND | — | — | GND plane | — |
| 13 | GND | — | — | GND shield CH2/CH3 | — |
| 14 | V1P8 | LP5912 1.8V | In | Analog ref / low-noise | Power |
| 15 | ADF_CH3P | ADC1 AINC+ | Out | ADAR2004 #2 IF Ch3+ (RX-EMVS 2 Hx) | Analog |
| 16 | V1P8 | LP5912 1.8V | In | Analog ref / low-noise | Power |
| 17 | ADF_CH3N | ADC1 AINC− | Out | ADAR2004 #2 IF Ch3− (RX-EMVS 2 Hx) | Analog |
| 18 | EXP_SPI_SCK | PC10 SPI3_SCK | In | Expansion periph CLK | SPI3 |
| 19 | GND | — | — | GND shield CH3/CH4 | — |
| 20 | EXP_SPI_MISO | PC11 SPI3_MISO | Out | Expansion periph DOUT | SPI3 |
| 21 | ADF_CH4P | ADC1 AIND+ | Out | ADAR2004 #2 IF Ch4+ (RX-EMVS 2 Hy) | Analog |
| 22 | EXP_SPI_MOSI | PC12 SPI3_MOSI | In | Expansion periph DIN | SPI3 |
| 23 | ADF_CH4N | ADC1 AIND− | Out | ADAR2004 #2 IF Ch4− (RX-EMVS 2 Hy) | Analog |
| 24 | EXP_SPI_NCS | PA15 SPI3_NSS | In | Expansion periph CS (active low) | SPI3 |
| 25 | GND | — | — | GND shield CH4/CH5 | — |
| 26 | CS_IO_EXP | PA11 GPIO | In | MAX7301 CS (I/O expander) | SPI6 |
| 27 | ADF_CH5P | ADC2 AINA+ | Out | ADAR2004 #1 IF Ch1+ (Ex) | Analog |
| 28 | GND | — | — | GND plane | — |
| 29 | ADF_CH5N | ADC2 AINA− | Out | ADAR2004 #1 IF Ch1− (Ex) | Analog |
| 30 | RF_SPI_SCK | SPI6_SCK (V10) | In | MAX7301 + all RF SCLK | SPI6 |
| 31 | GND | — | — | GND shield CH5/CH6 | — |
| 32 | RF_SPI_MOSI | SPI6_MOSI (T7) | In | MAX7301 DIN + all RF SDI | SPI6 |
| 33 | ADF_CH6P | ADC2 AINB+ | Out | ADAR2004 #1 IF Ch2+ (Ey) | Analog |
| 34 | RF_SPI_MISO | SPI6_MISO (W15) | Out | MAX7301 DOUT + RF SDO | SPI6 |
| 35 | ADF_CH6N | ADC2 AINB− | Out | ADAR2004 #1 IF Ch2− (Ey) | Analog |
| 36 | GND | — | — | GND plane | — |
| 37 | GND | — | — | GND shield CH6/CH7 | — |
| 38 | TXDATA_1 | PN12 GPIO | In | ADF4159 #1 TXDATA | GPIO |
| 39 | ADF_CH7P | ADC2 AINC+ | Out | ADAR2004 #1 IF Ch3+ (Hx) | Analog |
| 40 | TXDATA_2 | PA4 GPIO | In | ADF4159 #2 TXDATA | GPIO |
| 41 | ADF_CH7N | ADC2 AINC− | Out | ADAR2004 #1 IF Ch3− (Hx) | Analog |
| 42 | BPSK_GATE_1 | PG11 GPIO | In | BSS138 #1 gate | GPIO |
| 43 | GND | — | — | GND shield CH7/CH8 | — |
| 44 | BPSK_GATE_2 | PG2 GPIO | In | BSS138 #2 gate | GPIO |
| 45 | ADF_CH8P | ADC2 AIND+ | Out | ADAR2004 #1 IF Ch4+ (Hy) | Analog |
| 46 | MRST | PB12 GPIO | In | ADAR2001+2004×2 MRST (tied) | ADAR |
| 47 | ADF_CH8N | ADC2 AIND− | Out | ADAR2004 #1 IF Ch4− (Hy) | Analog |
| 48 | GND | — | — | GND plane | — |
| 49 | GND | — | — | GND shield CH8/CH9 | — |
| 50 | TxADV | PA10 GPIO | In | ADAR2001 TxADV (Rev E: was RxRST) | ADAR |
| 51 | ADF_CH9P | ADC3 AINA+ | Out | ADF5904 Ch A IF+ (Main RX Beam 1) | Analog |
| 52 | CNV_MASTER | TIM2_CH2 (W12) | In | AD7380-4 CNV (dig board) | Timer |
| 53 | ADF_CH9N | ADC3 AINA− | Out | ADF5904 Ch A IF− (Main RX Beam 1) | Analog |
| 54 | RxADV | PG15 GPIO | In | ADAR2004 #1+#2 RxADV (tied) | ADAR |
| 55 | GND | — | — | GND shield CH9/CH10 | — |
| 56 | CHIRP_START | TIM2_CH3 (W10) | In | ADF4159 timing ref | Timer |
| 57 | ADF_CH10P | ADC3 AINB+ | Out | ADF5904 Ch B IF+ (Main RX Beam 2) | Analog |
| 58 | MADV | PG13 GPIO | In | ADAR2001+2004×2 MADV (tied) | ADAR |
| 59 | ADF_CH10N | ADC3 AINB− | Out | ADF5904 Ch B IF− (Main RX Beam 2) | Analog |
| 60 | GND | — | — | GND plane | — |

## 13.9  Rev E Pin Changes (ADAR2001 Integration)

Pin 50 (PA10): Was RxRST → Now TxADV. Routes to ADAR2001 TX state machine advance input for per-chirp polarization cycling (Ex → Ey → Hx → Hy). RxRST function relocated to MAX7301 port P21 (slow recovery, SPI-accessible, no per-chirp timing).

Pin 46 (PB12) MRST: Now tied 3-way on analog board to ADAR2001 MRST + ADAR2004 #1 MRST + ADAR2004 #2 MRST. Synchronized multiplier reset across all three parts.

Pin 58 (PG13) MADV: Now tied 3-way on analog board to ADAR2001 MADV + ADAR2004 #1 MADV + ADAR2004 #2 MADV. Synchronized frequency stepping.

MAX7301 P20: New — CS_ADAR2001. No connector pin consumed; CS generated locally on analog board via I/O expander.

MAX7301 P21: New — RxRST relocated from pin 50. Slow recovery function, no timing requirement.

MAX7301 P22: New — TxRST for ADAR2001 transmitter reset. Slow recovery only.

# 14.  Design Audit Findings — Rev E

Audit date: 2026-04-29. Performed against HW-RDR-001 Rev E, Cyclops Digital Expansion Connector ICD, and ADAR2001/ADAR2004/LMX2594/AD7380-4 datasheets. Findings are classified CRITICAL (must fix before layout), SIGNIFICANT (design risk or functional gap), or MODERATE (documentation, optimization, or future risk).

| # | Severity | Finding | Recommended Fix | Affected Section | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | CRITICAL | ADC remap complete. CH1-CH4 (ADC1) = EMVS Cell 2 (all 4 ch). CH5-CH8 (ADC2) = EMVS Cell 1. CH9-CH10 (ADC3) = Main beam. One ADC per function. Analog board trace routing only. | FIXED: Full analog board remap. ADC1 (CH1-4) = Cell 2 Ex/Ey/Hx/Hy. ADC3 (CH9-10) = Main beam Ch A/B. Each ADC dedicated to one source. No mixed DMA. Analog board routing only — digital board and connector unchanged. | §13.6, §13.8, Block diagram | CLOSED |
| 2 | CRITICAL | LMX2594 splitter topology mismatch. BOM and document specify LMX2594 RFoutA → 2-way Wilkinson (ADAR2004 #1 + #2); RFoutB → ADAR2001 direct (+3.5 dBm/port). Rev E requires 3 ports: ADAR2001 TX + ADAR2004 #1 + ADAR2004 #2. 2-way cannot serve 3 loads. | FIXED: LMX2594 RFoutA → existing 2-way Wilkinson → ADAR2004 #1 + #2 (+3.5 dBm/port, unchanged). RFoutB (was terminated) → ADAR2001 direct (+7 dBm). No 3-way splitter needed. BOM unchanged. | §3.3, §10, Block diagram, BOM | CLOSED |
| 3 | CRITICAL | Level shifting undocumented. FIXED: §13.2 and §13.4 now specify TXS0108E 3.3 V ↔ 1.8 V translation for all ADAR/LMX signals. 3.3 V-native chips (ADF, BSS138) identified as exempt. | FIXED: Level-shifting requirements documented in §13.2 (SPI6 bus) and §13.4 (per-chirp GPIOs). TXS0108E specified for ADAR2001/ADAR2004/LMX2594 signals (1.8 V). ADF4159/ADF5901/ADF5904/BSS138 signals stay at 3.3 V (no translation). | §13.2, §13.4, §13.7 | CLOSED |
| 4 | SIGNIFICANT | ADAR2001 TXEN pin not allocated. FIXED: Assigned to MAX7301 P23 (active high, 10k pull-down). PA off at boot; firmware enables during EMVS TX dwells only. | FIXED: MAX7301 P23 = ADAR2001_TXEN. Added to §13.7 port table. 10k pull-down for safe boot. §13.7 EMVS_EN note updated. | §13.7, §4.3 | CLOSED |
| 5 | SIGNIFICANT | VBATT dropout at end-of-discharge. FIXED: §13.1 now specifies ultra-low-dropout LDO (TLV75533, 55 mV) or buck-boost (TPS63001) for analog board 3.3 V rail. 2.5 V EMVS rail has adequate headroom. | FIXED: Ultra-low-dropout LDO or buck-boost specified in §13.1. TLV75533 (55 mV dropout, 500 mA) recommended as default. | §13.1, §10 | CLOSED |
| 6 | SIGNIFICANT | Floating ADC inputs when EMVS off. FIXED: §13.6 specifies 100 kΩ bias resistors to VREF/2 on each diff leg on analog board. Firmware alternative: halt ADC sampling before EMVS_EN deassert. | FIXED: 100 kΩ bias resistors to VREF/2 specified in §13.6. Firmware halt alternative documented. | §13.6, §7.8 | CLOSED |
| 7 | SIGNIFICANT | ADAR2001 fault output not routed. FIXED: Assigned to MAX7301 P24 (input, 56k pull-up). Firmware polls in housekeeping loop for BIT. §13.7 port table updated. | FIXED: MAX7301 P24 = ADAR2001_FAULT. Added to §13.7 port table. 56k pull-up. Firmware housekeeping poll. | §13.7 | CLOSED |
| 8 | MODERATE | Pin 52 (CNV_MASTER) recoverable. RESOLVED: Channel gap fixed by analog board ADC remap (finding #1). Pin 52 no longer needed for CH11. Retained as CNV_MASTER sync reference — useful for analog board timing verification during bring-up. | RESOLVED: No reassignment needed. ADC remap solved channel gap. Pin 52 retained as sync reference for bring-up. Available for future reuse if needed. | §13.5, §13.8 | CLOSED |
| 9 | MODERATE | Residual SPI1 references. Several table entries in §7 still show "SPI1 shared bus" in CS notes (e.g., CS_ADAR2004_2 at PF1). Should reference SPI6/RF_SPI per connector ICD. | Global search-and-replace: "SPI1 shared bus" to "SPI6 (RF_SPI) shared bus" in §7 tables. | §7.2, §7.5 | CLOSED |
| 10 | MODERATE | CHIRP_START (pin 56) clarified. FIXED: TIM2_CH3 hardware chirp-rate reference pulse. Used for LMX2594 ramp sync, TxADV timing ref, and scope trigger during bring-up. Not an ADF4159 trigger (ramps are internal). Retained as-is. | FIXED: Function clarified — hardware chirp-rate timing reference from TIM2_CH3. Used for LMX2594 ramp sync, TxADV timing, and bring-up trigger. Not a candidate for reassignment. | §13.5, §13.8 | CLOSED |
| 11 | MODERATE | ADAR2001 RF output routing undocumented. FIXED: §13.8 signal routing summary now documents ADAR2001 PA-to-TX-EMVS traces as analog-board-internal. 100 ohm diff impedance, 10 mm max length, inner-layer shielded routing, 3 mm keep-out from K-band TX specified. | FIXED: Routing constraints added to §13.8. 100 ohm diff, 10 mm max, inner layer, 3 mm keep-out from HMC1131. | §13 | CLOSED |
| 12 | MODERATE | LO power budget. RESOLVED by finding #2: No 3-way split exists. LMX2594 RFoutA feeds 2-way Wilkinson at +3.5 dBm/port to ADAR2004 x2 (unchanged from Rev D). RFoutB feeds ADAR2001 direct at +7 dBm. All ports well above -20 dBm minimum. No power degradation vs Rev D. | RESOLVED: +3.5 dBm/port (ADAR2004) and +7 dBm (ADAR2001) — same or better than Rev D. Standard bench verification during bring-up; no special characterization needed. | §3.3, §10 | CLOSED |

## 14.1  Resolution Priority

1. Fix #1 (EMVS Cell 2 channels) — architectural; determines connector pin assignment and ADC routing. Evaluate reclaiming pins 52/56 for CH11/CH12 before committing analog board layout.

2. Fix #2 (LMX2594 splitter) — schematic change, affects BOM and PCB. RESOLVED: LMX2594 dual output (RFoutA 2-way, RFoutB direct). No layout impact.

3. Fix #3 (level shifting) — schematic addition. TXS0108E placement and routing must be in analog board layout.

4. Fix #4 (ADAR2001 TXEN) — one MAX7301 port assignment + one pull-down resistor. Trivial schematic change.

5. Fixes #5–#7 — power and reliability. Can be addressed during schematic review without blocking layout.

6. Fixes #8–#12 — documentation and characterization. Can be addressed in parallel with bring-up.

## 13.10  ADAR2001 IC Pinout — Analog Board Connections (40-Terminal LGA, 6x6 mm)

Complete pin-by-pin mapping from ADAR2001 datasheet (Rev A, Table 6) to Cyclops Pro Rev E analog board connections. Package: CC-40-7, 40-terminal LGA. All digital pins operate at 1.8 V logic (VREG output). SPI and state machine pins require TXS0108E level shifting from 3.3 V connector domain.

| Pin | Mnemonic | Type | Cyclops Pro Connection | Level | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | VPOS1 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap to GND |
| 2 | GND1 | GND | Analog GND plane | — | Low impedance |
| 3 | GND2 | GND | Analog GND plane | — |  |
| 4 | RFIN | RF IN | LMX2594 RFoutB (direct, no splitter) | — | 50 ohm, 2.5-10 GHz, AC-coupled internally. +7 dBm from LMX2594. |
| 5 | GND3 | GND | Analog GND plane | — |  |
| 6 | VREG | PWR OUT | Connect directly to pin 7 (VPOS2). 1 uF to GND. | 1.8 V | Internal LDO output. Also supplies VCCA for TXS0108E level shifter. |
| 7 | VPOS2 | PWR | Connect directly to pin 6 (VREG) | 1.8 V | Digital supply. Do NOT connect to external 1.8 V. |
| 8 | TxADV | DIG IN | Connector pin 50 (PA10) via TXS0108E | 1.8 V | Per-chirp pol cycling. 3 ns min pulse. Direct GPIO from STM32. |
| 9 | TxRST | DIG IN | MAX7301 P22 via TXS0108E | 1.8 V | TX state machine reset. Slow recovery only. |
| 10 | MADV | DIG IN | Connector pin 58 (PG13) via TXS0108E, tied with ADAR2004 x2 | 1.8 V | Multiplier advance. 3-way shared. 3 ns min pulse. |
| 11 | MRST | DIG IN | Connector pin 46 (PB12) via TXS0108E, tied with ADAR2004 x2 | 1.8 V | Multiplier reset. 3-way shared. |
| 12 | SCLK | DIG IN | RF_SPI_SCK (connector pin 30) via TXS0108E | 1.8 V | SPI clock. Max 40 MHz write, 15 MHz read. |
| 13 | SDIO | DIG I/O | RF_SPI_MOSI (connector pin 32) via TXS0108E | 1.8 V | 3-wire mode: bidirectional. 4-wire: input only. |
| 14 | CS | DIG IN | MAX7301 P20 via TXS0108E | 1.8 V | Active low. 200 kohm pull-up to 1.8 V (VREG). |
| 15 | SDO | DIG OUT | RF_SPI_MISO (connector pin 34) via TXS0108E | 1.8 V | 4-wire mode readback. Directly or tri-state on shared bus. |
| 16 | GND4 | GND | Analog GND plane | — |  |
| 17 | GND5 | GND | Analog GND plane | — |  |
| 18 | RFOUT4+ | RF OUT | TX-EMVS 1 cell Hy+ (diff pair) | — | 100 ohm diff, AC-coupled internally. Route on inner layer. |
| 19 | RFOUT4- | RF OUT | TX-EMVS 1 cell Hy- (diff pair) | — | Match length to RFOUT4+. Max 10 mm. |
| 20 | GND6 | GND | Analog GND plane | — |  |
| 21 | VPOS3 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap to GND |
| 22 | GND7 | GND | Analog GND plane | — |  |
| 23 | RFOUT3+ | RF OUT | TX-EMVS 1 cell Hx+ (diff pair) | — | 100 ohm diff |
| 24 | RFOUT3- | RF OUT | TX-EMVS 1 cell Hx- (diff pair) | — | Match length to RFOUT3+ |
| 25 | GND8 | GND | Analog GND plane | — |  |
| 26 | GND9 | GND | Analog GND plane | — |  |
| 27 | GND10 | GND | Analog GND plane | — |  |
| 28 | RFOUT2- | RF OUT | TX-EMVS 1 cell Ey- (diff pair) | — | 100 ohm diff |
| 29 | RFOUT2+ | RF OUT | TX-EMVS 1 cell Ey+ (diff pair) | — | Match length to RFOUT2- |
| 30 | GND11 | GND | Analog GND plane | — |  |
| 31 | VPOS4 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap to GND |
| 32 | GND12 | GND | Analog GND plane | — |  |
| 33 | RFOUT1- | RF OUT | TX-EMVS 1 cell Ex- (diff pair) | — | 100 ohm diff |
| 34 | RFOUT1+ | RF OUT | TX-EMVS 1 cell Ex+ (diff pair) | — | Match length to RFOUT1- |
| 35 | GND13 | GND | Analog GND plane | — |  |
| 36 | GND14 | GND | Analog GND plane | — |  |
| 37 | GND15 | GND | Analog GND plane | — |  |
| 38 | GND16 | GND | Analog GND plane | — |  |
| 39 | GND17 | GND | Analog GND plane | — |  |
| 40 | VPOS5 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap to GND |
| EPAD | EPAD | GND | Analog GND plane via thermal via array | — | Critical: 450 mW dissipation. Min 9 vias under pad. |

RF Output Channel to EMVS Element Mapping:

RFOUT1+/- (pins 34/33) = Ex dipole. RFOUT2+/- (pins 29/28) = Ey dipole. RFOUT3+/- (pins 23/24) = Hx loop. RFOUT4+/- (pins 18/19) = Hy loop. All four diff pairs must be length-matched to within 0.1 mm. Route on inner layer with continuous ground plane above and below. Maintain 3 mm keep-out from HMC1131 K-band TX output trace and via wall.

Decoupling Summary: 4x VPOS pins each get 100 pF + 10 nF. One shared 1 uF for the 2.5 V rail. VREG/VPOS2 get 1 uF. Total: 9x 100 pF, 4x 10 nF, 2x 1 uF. Place all caps within 2 mm of their respective pins.

TXEN Control: ADAR2001 does not have a dedicated TXEN pin. PA enable is controlled per-channel via SPI registers (TX_EN1/TX_EN2, Register 0x050/0x051) or via the TX state machine (TxADV/TxRST). Global PA shutdown is achieved by writing TX sleep mode via SPI. MAX7301 P23 (ADAR2001_TXEN) gates power to the ADAR2001 PA supply via an external PFET or load switch, providing hardware-level PA disable independent of SPI state.

## 13.11  ADAR2004 #1 IC Pinout — RX-EMVS Cell 1 (48-Terminal LGA, 7x7 mm)

ADAR2004 #1 receives from RX-EMVS Cell 1 (top-right corner). IF outputs route to AD7380-4 #2 (ADC2) via connector CH5-CH8. LO from LMX2594 RFoutA via 2-way Wilkinson. CS from MAX7301 P9. All digital pins 1.8 V via TXS0108E level shifter.

| Pin | Mnemonic | Type | Cyclops Pro Connection | Level | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | VPOS1 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap |
| 2 | GND1 | GND | Analog GND plane | - |  |
| 3 | RFIN2- | RF IN | RX-EMVS 1 Ey- (top-right) | - | 100 ohm diff, 10-40 GHz, AC-coupled |
| 4 | RFIN2+ | RF IN | RX-EMVS 1 Ey+ (top-right) | - |  |
| 5 | GND2 | GND | Analog GND plane | - |  |
| 6 | GND3 | GND | Analog GND plane | - |  |
| 7 | GND4 | GND | Analog GND plane | - |  |
| 8 | GND5 | GND | Analog GND plane | - |  |
| 9 | GND6 | GND | Analog GND plane | - |  |
| 10 | RFIN3+ | RF IN | RX-EMVS 1 Hx+ (top-right) | - | 100 ohm diff |
| 11 | RFIN3- | RF IN | RX-EMVS 1 Hx- (top-right) | - |  |
| 12 | GND7 | GND | Analog GND plane | - |  |
| 13 | VPOS2 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap |
| 14 | GND8 | GND | Analog GND plane | - |  |
| 15 | RFIN4+ | RF IN | RX-EMVS 1 Hy+ (top-right) | - | 100 ohm diff |
| 16 | RFIN4- | RF IN | RX-EMVS 1 Hy- (top-right) | - |  |
| 17 | GND9 | GND | Analog GND plane | - |  |
| 18 | GND10 | GND | Analog GND plane | - |  |
| 19 | IFOUT4+ | IF OUT | Connector CH8P (pin 45) -> ADC2 AIND+ (Hy) | - | 100 ohm diff, DC-coupled, VOCM=0.8 V |
| 20 | IFOUT4- | IF OUT | Connector CH8N (pin 47) -> ADC2 AIND- (Hy) | - |  |
| 21 | IFOUT3+ | IF OUT | Connector CH7P (pin 39) -> ADC2 AINC+ (Hx) | - | 100 ohm diff |
| 22 | IFOUT3- | IF OUT | Connector CH7N (pin 41) -> ADC2 AINC- (Hx) | - |  |
| 23 | GND11 | GND | Analog GND plane | - |  |
| 24 | SDO | DIG OUT | RF_SPI_MISO (pin 34) via TXS0108E | 1.8 V | 4-wire readback |
| 25 | CS | DIG IN | MAX7301 P9 via TXS0108E | 1.8 V | Active low. 200 kohm pull-up to VREG. |
| 26 | SDIO | DIG I/O | RF_SPI_MOSI (pin 32) via TXS0108E | 1.8 V | 3-wire bidir or 4-wire input |
| 27 | SCLK | DIG IN | RF_SPI_SCK (pin 30) via TXS0108E | 1.8 V | Max 40 MHz write, 15 MHz read |
| 28 | RxRST | DIG IN | MAX7301 P21 via TXS0108E (Rev E: relocated) | 1.8 V | Rx state machine reset. Slow recovery. |
| 29 | RxADV | DIG IN | Connector pin 54 (PG15) via TXS0108E | 1.8 V | Rx advance. Tied between both ADAR2004s. |
| 30 | MRST | DIG IN | Connector pin 46 (PB12) via TXS0108E, 3-way shared | 1.8 V | Mult reset. Tied with ADAR2001. |
| 31 | MADV | DIG IN | Connector pin 58 (PG13) via TXS0108E, 3-way shared | 1.8 V | Mult advance. Tied with ADAR2001. |
| 32 | VREG | PWR OUT | Connect directly to pin 33 (VPOS3). 1 uF to GND. | 1.8 V | Internal LDO. Also feeds TXS0108E VCCA. |
| 33 | VPOS3 | PWR | Connect directly to pin 32 (VREG) | 1.8 V | Digital supply. |
| 34 | GND12 | GND | Analog GND plane | - |  |
| 35 | LOIN | RF IN | LMX2594 RFoutA via 2-way Wilkinson (+3.5 dBm) | - | 50 ohm, 2.4-10.1 GHz, nom -20 dBm |
| 36 | GND13 | GND | Analog GND plane | - |  |
| 37 | GND14 | GND | Analog GND plane | - |  |
| 38 | VPOS4 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap |
| 39 | GND15 | GND | Analog GND plane | - |  |
| 40 | IFOUT2- | IF OUT | Connector CH6N (pin 35) -> ADC2 AINB- (Ey) | - | 100 ohm diff |
| 41 | IFOUT2+ | IF OUT | Connector CH6P (pin 33) -> ADC2 AINB+ (Ey) | - |  |
| 42 | IFOUT1- | IF OUT | Connector CH5N (pin 29) -> ADC2 AINA- (Ex) | - | 100 ohm diff |
| 43 | IFOUT1+ | IF OUT | Connector CH5P (pin 27) -> ADC2 AINA+ (Ex) | - |  |
| 44 | GND16 | GND | Analog GND plane | - |  |
| 45 | GND17 | GND | Analog GND plane | - |  |
| 46 | RFIN1- | RF IN | RX-EMVS 1 Ex- (top-right) | - | 100 ohm diff |
| 47 | RFIN1+ | RF IN | RX-EMVS 1 Ex+ (top-right) | - |  |
| 48 | GND18 | GND | Analog GND plane | - |  |
| EPAD | EPAD | GND | Analog GND via thermal via array | - | 910 mW dissipation. Min 16 vias under pad. |

ADAR2004 #1 RF/IF Channel Mapping: RFIN1 (pins 47/46) = Ex. RFIN2 (pins 4/3) = Ey. RFIN3 (pins 10/11) = Hx. RFIN4 (pins 15/16) = Hy. IFOUT1 (pins 43/42) = Ex -> CH5. IFOUT2 (pins 41/40) = Ey -> CH6. IFOUT3 (pins 21/22) = Hx -> CH7. IFOUT4 (pins 19/20) = Hy -> CH8.

## 13.12  ADAR2004 #2 IC Pinout — RX-EMVS Cell 2 (48-Terminal LGA, 7x7 mm)

ADAR2004 #2 receives from RX-EMVS Cell 2 (bottom-left corner). IF outputs route to AD7380-4 #1 (ADC1) via connector CH1-CH4 (Rev E analog board remap). LO from LMX2594 RFoutA via 2-way Wilkinson. CS from MAX7301 P10. All digital pins 1.8 V via TXS0108E level shifter.

| Pin | Mnemonic | Type | Cyclops Pro Connection | Level | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | VPOS1 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap |
| 2 | GND1 | GND | Analog GND plane | - |  |
| 3 | RFIN2- | RF IN | RX-EMVS 2 Ey- (bottom-left) | - | 100 ohm diff, 10-40 GHz, AC-coupled |
| 4 | RFIN2+ | RF IN | RX-EMVS 2 Ey+ (bottom-left) | - |  |
| 5 | GND2 | GND | Analog GND plane | - |  |
| 6 | GND3 | GND | Analog GND plane | - |  |
| 7 | GND4 | GND | Analog GND plane | - |  |
| 8 | GND5 | GND | Analog GND plane | - |  |
| 9 | GND6 | GND | Analog GND plane | - |  |
| 10 | RFIN3+ | RF IN | RX-EMVS 2 Hx+ (bottom-left) | - | 100 ohm diff |
| 11 | RFIN3- | RF IN | RX-EMVS 2 Hx- (bottom-left) | - |  |
| 12 | GND7 | GND | Analog GND plane | - |  |
| 13 | VPOS2 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap |
| 14 | GND8 | GND | Analog GND plane | - |  |
| 15 | RFIN4+ | RF IN | RX-EMVS 2 Hy+ (bottom-left) | - | 100 ohm diff |
| 16 | RFIN4- | RF IN | RX-EMVS 2 Hy- (bottom-left) | - |  |
| 17 | GND9 | GND | Analog GND plane | - |  |
| 18 | GND10 | GND | Analog GND plane | - |  |
| 19 | IFOUT4+ | IF OUT | Connector CH4P (pin 21) -> ADC1 AIND+ (Hy) | - | 100 ohm diff, DC-coupled, VOCM=0.8 V |
| 20 | IFOUT4- | IF OUT | Connector CH4N (pin 23) -> ADC1 AIND- (Hy) | - |  |
| 21 | IFOUT3+ | IF OUT | Connector CH3P (pin 15) -> ADC1 AINC+ (Hx) | - | 100 ohm diff |
| 22 | IFOUT3- | IF OUT | Connector CH3N (pin 17) -> ADC1 AINC- (Hx) | - |  |
| 23 | GND11 | GND | Analog GND plane | - |  |
| 24 | SDO | DIG OUT | RF_SPI_MISO (pin 34) via TXS0108E | 1.8 V | 4-wire readback |
| 25 | CS | DIG IN | MAX7301 P10 via TXS0108E | 1.8 V | Active low. 200 kohm pull-up to VREG. |
| 26 | SDIO | DIG I/O | RF_SPI_MOSI (pin 32) via TXS0108E | 1.8 V | 3-wire bidir or 4-wire input |
| 27 | SCLK | DIG IN | RF_SPI_SCK (pin 30) via TXS0108E | 1.8 V | Max 40 MHz write, 15 MHz read |
| 28 | RxRST | DIG IN | MAX7301 P21 via TXS0108E (Rev E: relocated) | 1.8 V | Rx state machine reset. Slow recovery. |
| 29 | RxADV | DIG IN | Connector pin 54 (PG15) via TXS0108E | 1.8 V | Rx advance. Tied between both ADAR2004s. |
| 30 | MRST | DIG IN | Connector pin 46 (PB12) via TXS0108E, 3-way shared | 1.8 V | Mult reset. Tied with ADAR2001. |
| 31 | MADV | DIG IN | Connector pin 58 (PG13) via TXS0108E, 3-way shared | 1.8 V | Mult advance. Tied with ADAR2001. |
| 32 | VREG | PWR OUT | Connect directly to pin 33 (VPOS3). 1 uF to GND. | 1.8 V | Internal LDO. Also feeds TXS0108E VCCA. |
| 33 | VPOS3 | PWR | Connect directly to pin 32 (VREG) | 1.8 V | Digital supply. |
| 34 | GND12 | GND | Analog GND plane | - |  |
| 35 | LOIN | RF IN | LMX2594 RFoutA via 2-way Wilkinson (+3.5 dBm) | - | 50 ohm, 2.4-10.1 GHz, nom -20 dBm |
| 36 | GND13 | GND | Analog GND plane | - |  |
| 37 | GND14 | GND | Analog GND plane | - |  |
| 38 | VPOS4 | PWR | EMVS 2.5 V LDO output | 2.5 V | 100 pF + 10 nF decap |
| 39 | GND15 | GND | Analog GND plane | - |  |
| 40 | IFOUT2- | IF OUT | Connector CH2N (pin 11) -> ADC1 AINB- (Ey) | - | 100 ohm diff |
| 41 | IFOUT2+ | IF OUT | Connector CH2P (pin 9) -> ADC1 AINB+ (Ey) | - |  |
| 42 | IFOUT1- | IF OUT | Connector CH1N (pin 5) -> ADC1 AINA- (Ex) | - | 100 ohm diff |
| 43 | IFOUT1+ | IF OUT | Connector CH1P (pin 3) -> ADC1 AINA+ (Ex) | - |  |
| 44 | GND16 | GND | Analog GND plane | - |  |
| 45 | GND17 | GND | Analog GND plane | - |  |
| 46 | RFIN1- | RF IN | RX-EMVS 2 Ex- (bottom-left) | - | 100 ohm diff |
| 47 | RFIN1+ | RF IN | RX-EMVS 2 Ex+ (bottom-left) | - |  |
| 48 | GND18 | GND | Analog GND plane | - |  |
| EPAD | EPAD | GND | Analog GND via thermal via array | - | 910 mW dissipation. Min 16 vias under pad. |

ADAR2004 #2 RF/IF Channel Mapping (Rev E remap): RFIN1 (pins 47/46) = Ex. RFIN2 (pins 4/3) = Ey. RFIN3 (pins 10/11) = Hx. RFIN4 (pins 15/16) = Hy. IFOUT1 (pins 43/42) = Ex -> CH1. IFOUT2 (pins 41/40) = Ey -> CH2. IFOUT3 (pins 21/22) = Hx -> CH3. IFOUT4 (pins 19/20) = Hy -> CH4. Note: CH1-CH4 were originally ADF5904 Ch A-D; reassigned to EMVS Cell 2 in Rev E analog board remap.

Shared signals between ADAR2004 #1 and #2: MADV (pin 31, connector pin 58), MRST (pin 30, connector pin 46), RxADV (pin 29, connector pin 54) are tied on the analog board. RxRST (pin 28) is driven from MAX7301 P21 (shared). SCLK/SDIO/SDO share the SPI6 bus; only CS differs (P9 vs P10). LO input (pin 35) shares the Wilkinson splitter output. Both parts are powered from the same EMVS 2.5 V LDO gated by EMVS_EN (P14). Both VREG outputs (pin 32) can feed a common 1.8 V node for the TXS0108E VCCA, but keep decoupling local to each IC.

## 13.13  RF Matching Networks & Impedance Routing (Analog Board)

All RF matching networks are placed on the analog board, at the ADAR IC pads. The antenna board is a passive structure carrying only impedance-controlled differential traces with no discrete components. At 10-40 GHz a quarter wavelength is 0.75-3 mm, so matching elements must be within this distance of the device being matched. Placing matching on the antenna board would interpose the board-to-board transition impedance, making tuning impractical.

### 13.13.1  ADAR2001 TX Output Matching

| IC Pins | Mnemonic | EMVS Element | Z_diff | Power | Band |
| --- | --- | --- | --- | --- | --- |
| 34 / 33 | RFOUT1+/- | Ex dipole | 100 ohm | +5 dBm | 10-40 GHz |
| 29 / 28 | RFOUT2+/- | Ey dipole | 100 ohm | +5 dBm | 10-40 GHz |
| 23 / 24 | RFOUT3+/- | Hx loop | 100 ohm | +5 dBm | 10-40 GHz |
| 18 / 19 | RFOUT4+/- | Hy loop | 100 ohm | +5 dBm | 10-40 GHz |

Matching topology: Pi-match footprint on each differential pair — one 0201 series element + two 0201 shunt elements to GND, placed within 0.5 mm of the ADAR2001 output pad. Populate with 0 ohm series / open shunt (through) as default; tune on bench with VNA measuring S11 at the ADAR2001 output through the full 10-40 GHz band. AC coupling is internal to the ADAR2001 — no external DC blocking caps needed.

Trace geometry: 100 ohm differential on RO4350B 0.508 mm substrate. Trace width ~0.10 mm, gap ~0.15 mm on 0.10 mm dielectric. Route on inner layer with continuous GND planes above and below. All 8 lines (4 diff pairs) length-matched to within 0.1 mm (1.2 ps skew at 40 GHz). Maximum trace length from ADAR2001 pad to TX-EMVS 1 cell feed via: 10 mm. Maintain 3 mm keep-out from HMC1131 K-band TX output trace and from the TX1/TX-EMVS via wall boundary.

### 13.13.2  ADAR2004 RX Input Matching

| IC Pins | Mnemonic | EMVS Element | Z_diff | RX Power | Band |
| --- | --- | --- | --- | --- | --- |
| 47 / 46 | RFIN1+/- | Ex dipole | 100 ohm | -40 to -20 dBm | 10-40 GHz |
| 4 / 3 | RFIN2+/- | Ey dipole | 100 ohm | -40 to -20 dBm | 10-40 GHz |
| 10 / 11 | RFIN3+/- | Hx loop | 100 ohm | -40 to -20 dBm | 10-40 GHz |
| 15 / 16 | RFIN4+/- | Hy loop | 100 ohm | -40 to -20 dBm | 10-40 GHz |

Same pi-match topology as TX: 0201 series + 2x shunt within 0.5 mm of each ADAR2004 input pad. Start with 0 ohm series / open shunt. Tune per cell after bench S11 measurement. The EMVS cell probe feed may be close to 100 ohm natively — matching components may not be needed, but pads must be in the layout for production tuning flexibility. Same trace geometry and length-matching rules as §13.13.1.

ADAR2004 #1: RF traces from RX-EMVS 1 cell (top-right corner of analog board). ADAR2004 #2: RF traces from RX-EMVS 2 cell (bottom-left corner). Route each set of 4 diff pairs as a matched group. Keep RF input traces away from IF output traces (different frequency domains) with minimum 1 mm separation or a GND pour between them.

### 13.13.3  LMX2594 LO Distribution

| Output | Destination | Impedance | Power | Frequency | Topology |
| --- | --- | --- | --- | --- | --- |
| RFoutA (pin 11) | ADAR2004 #1 + #2 LOIN | 50 ohm SE | +7 dBm total | 6 GHz | 2-way Wilkinson splitter, +3.5 dBm/port |
| RFoutB (pin TBD) | ADAR2001 RFIN (pin 4) | 50 ohm SE | +7 dBm direct | 6 GHz | Point-to-point, no splitter |

RFoutA Wilkinson splitter: Equal arm lengths, 70.7 ohm quarter-wave transmission line sections at 6 GHz (lambda/4 ~ 6.6 mm on RO4350B), 100 ohm thin-film isolation resistor at the split junction. Phase match both arms to within 5 degrees at 6 GHz for coherent x4 multiplication in both ADAR2004s. 50 ohm microstrip for all LO traces. Place splitter within 15 mm of LMX2594 output.

RFoutB direct trace: 50 ohm microstrip from LMX2594 RFoutB pad to ADAR2001 RFIN (pin 4). Point-to-point, no splitter. Keep under 20 mm total length. AC coupling caps at both ends (100 pF, 0201).

### 13.13.4  Antenna Board RF Transition

The antenna board carries 100 ohm differential traces from each EMVS cell probe feed point to the board-to-board RF transition (via array or press-fit connector). No discrete matching components on the antenna board. The antenna board is a passive structure — if the cell probe impedance deviates from 100 ohm, compensation is applied on the analog board matching network (§13.13.1 / §13.13.2).

Transition requirements: Ground vias flanking each differential pair at the board-to-board interface. Minimum 2 ground vias per diff pair (one on each side). Via diameter 0.2 mm, pad 0.4 mm. Characterize insertion loss and return loss of the transition with a VNA on a test coupon before committing to production. Target: S21 better than -1 dB, S11 better than -15 dB across 10-40 GHz.

Antenna board substrate: RO4350B, 0.508 mm thickness, F.Cu only for EMVS cell elements. 100 ohm differential trace geometry matches analog board stackup. Maintain ground plane continuity under all RF traces on both boards.

### 13.13.5  RF IC Decoupling Summary

| IC | Supply | Per-Pin Decap | Rail Bulk | Placement |
| --- | --- | --- | --- | --- |
| ADAR2001 | VPOS1/3/4/5 (2.5V, 4 pins) | 100 pF + 10 nF each | 1 uF shared | Within 2 mm of each VPOS pin |
| ADAR2001 | VREG/VPOS2 (1.8V, 2 pins) | — | 1 uF | Direct bridge pin 6→7, cap at pin 7 |
| ADAR2004 x2 | VPOS1/2/4 (2.5V, 3 pins each) | 100 pF + 10 nF each | 1 uF shared per IC | Within 2 mm of each VPOS pin |
| ADAR2004 x2 | VREG/VPOS3 (1.8V, 2 pins each) | — | 1 uF per IC | Direct bridge pin 32→33 |
| LMX2594 | VCC (3.3V, 4 pins) | 100 nF each | 10 uF shared | Within 2 mm of each VCC pin |
| MAX7301 | VCC (3.3V, 1 pin) | 100 nF | 10 uF | Within 2 mm |

End of document -- HW-RDR-001 Rev E.

# Appendix — EMVS Cell Rev D Topology Update

Added: 2026-04-29  ·  Author: Ocupoint Engineering  ·  Companion: EMVS_Cell_RevD_Verification_RevA.docx

## Background

The original concentric EMVS cell layout placed the Hy planar loop encircling the Ex/Ey crossed dipoles, with the Hx loop crossing Hy near y = -0.71 mm.  This produced a physical interlock between Hx and Hy that gave 4.8 dB Hx<->Hy isolation in 2.5D MoM (target >= 20 dB per HW-RDR-001 Rev D §4).  Bench would have failed all four H-loop reception tests.

## Rev D Topology

The Rev D layout removes the Hx<->Hy interlock entirely:

• Hx is now a tall narrow loop on the cell's LEFT edge (centre at -1.40, 0; outer 0.36 × 2.50 mm).

• Hy is a planar loop in the LOWER-RIGHT quadrant (centre at +0.80, -0.95; outer 1.40 × 1.00 mm).

• Ex/Ey crossed dipoles are shifted to (-0.05, +0.40) to balance Ex<->Hx and Ey<->Hy lateral gaps.

• Dipole feed gap bumped from 0.15 mm to 0.20 mm to recover X-band Ex<->Ey at the 25 dB threshold.

• Cell footprint unchanged at 3.16 × 3.16 mm; trace 0.10 mm; F.Cu only on RO4350B 0.508 mm.

## Verification — All 12 Paths PASS

MoM verification per analyze_emvs_revD_topology.m:

| Path | Description | Target [dB] | K-band 24.125 GHz | X-band 10 GHz (EMVS) | Result |
| --- | --- | --- | --- | --- | --- |
| Ex <-> Ey | orthogonal dipoles | 25 | 28.7 | 26.4 | PASS |
| Ex <-> Hx | co-plane orthogonal | 20 | 47.5 | 39.8 | PASS |
| Ey <-> Hy | co-plane orthogonal | 20 | 33.5 | 44.1 | PASS |
| Ex <-> Hy | cross-plane | 25 | 27.9 | 33.6 | PASS |
| Ey <-> Hx | cross-plane | 25 | 37.7 | 32.8 | PASS |
| Hx <-> Hy | two H-loops (was 4.8 dB) | 20 | 34.6 | 29.0 | PASS |
| — | overall result | — | 6/6 | 6/6 | PASS |

## Deliverables Updated

• KiCad footprint: cyclops_emvs_cell_revD.kicad_mod

• MATLAB MoM script: analyze_emvs_revD_topology.m

• Verification report: EMVS_Cell_RevD_Verification_RevA.docx

• Topology drawing: emvs_revD_topology.png

• S-parameter datafile: emvs_revD_S.mat

## Open Items

• Place the EMVS_Cell_RevD footprint on the analog board file (separate work item — not on the patch-array antenna board).

• Bench measure isolation on first fab once boards arrive — confirm Hx<->Hy >= 25 dB.

• If bench Hx<->Hy < 25 dB, candidate fix: shrink Hy to 1.20 × 1.00 (left wall from x=+0.10 to x=+0.30).

## Companion Antenna PCB Update

The patch-array antenna board has been forked rev5 -> rev6 with the following GND-fence and fab-process additions, captured in cyclops_narrow_antenna_rev6_dualband.kicad_pcb:

• TX1 right-wall GND pour (x=25.4-25.9, y=14.9-17.98) bridging the existing 4-via column.

• TX2 left-wall GND pour (x=54.10-54.60, y=31.9-35.09) — mirror of TX1.

• TX-EMVS 1 right-wall GND pour (x=22.50-23.60, y=31.90-34.80) bridging 6 vias in 2 columns.

• TX-EMVS 1 left-wall GND pour (x=56.40-57.45, y=14.10-18.10) bridging 10 vias in 2 columns.

• Edge-plating fab note added to F.SilkS — all four board edges to be plated to bridge F.Cu+B.Cu GND pours into a continuous Faraday wall.  Estimated +7 to +12 dB on the failing TX2 -> SA2-E7 X-band isolation path.